BEFORE THE WASHINGTON UTILITIES AND TRANSPORTATION COMMISSION

| In the Matter of the Petition of THE BURLINGTON NORTHERN SANTA FE RAILWAY COMPANY for Modification of Order Regulating the Speed of Passenger and Freight Trains in Auburn, Washington | DOCKET NO. TR-990655 AMENDMENT WAIVING PROVISION OF ORIGINAL ORDER REQUIRING MEDIAN BARRIERS ON THE EAST SIDE OF WEST MAIN STREET, AND ALSO ALLOWING HEAD END SPEED RESTRICTIONS |
| --- | --- |

# BACKGROUND

- On June 16, 2000, the Commission entered an order approving a request by the Burlington Northern Santa Fe Railway Company (BNSF) to modify train speed limits through the City of Auburn to the maximum speeds allowed by the Federal Railroad Administration, subject to certain safety improvements.  The safety improvements included the installation of median barriers on both approaches to the West Main Street highway-rail grade crossing.

# DISCUSSION

- Median barriers consist of a row of delineators down the centerline of a roadway that extend from the tip of a crossing gate arm to some point in the opposite direction of the tracks.  The purpose of the barriers is to prevent motorists from driving around lowered gates.  Ideally, median barriers should extend 100 feet or more; however, barriers as short as 50 feet are acceptable where longer installations would conflict with access to other roadways or driveways.

- After the original order was filed, the City of Auburn expressed concerns about the installation of median barriers at the West Main Street grade crossing, identified as DOT crossing 085655A.  The roadway intersection of West Main Street and B Street is located immediately adjacent to the east side of the grade crossing.  B Street is parallel to the BNSF tracks, with approximately 30 feet separating the two.  If median barriers were installed on the east side of West Main Street, the barriers would need to extend into the West Main Street/B Street roadway intersection in order to be effective.  Barriers that are too short (i.e. under 50 feet) are considered ineffective due to the room they leave for motorists to drive around both the median barriers and gates.  Installing a 50 foot barrier on the east side of the crossing would restrict turning movements for the West Main/B Street intersection to right-turn in and right-turn out only.  Traffic turning from B Street to West Main Street, as well as from West Main Street to B Street includes transit buses and trucks.  These vehicles typically have long wheel bases and require a wide turning radius.  Installing 50 foot median barriers on the east side of the crossing would likely result in some of these vehicles running over and damaging the barriers when making right turns onto West Main Street from B Street.

- On July 10, 2002, The City of Auburn filed a written request with the Commission to amend order TR-990655 to exclude the installation of median barriers on the east side of the crossing.  The request states that “a median installation on the east leg of the Main Street crossing creates geometric concerns for vehicles turning right at B Street.  Terminating a median in the middle of the B Street intersection is an unacceptable alternative for geometric reasons.”

- In addition to the City of Auburn’s request, the Burlington Northern Santa Fe Railway Company has requested that the original order be amended to specify that the speed restrictions set forth by the order be converted to a head-end restriction.  This would allow a train to increase speed as soon as the head end of a train passed a speed restriction boundary rather than waiting until the rear of the train passed the boundary.  An identical provision concerning a “head-end” restriction is stated in Commission order TR-990656, approving train speed increases through the City of Puyallup.  The City of Auburn concurred in writing that there are no safety concerns that preclude this change.

- Commission Staff investigated the parties’ requests and recommended that the original order be amended to reflect the requests.

- The Burlington Northern Santa Fe Railway Company does not object to the amendment as requested by the City of Auburn.  The City of Auburn does not object to the amendment requested by BNSF.

- This matter was brought before the Commission at its regularly scheduled open meeting on July 26, 2002.  The Commissioners have reviewed the request for amendment and have been advised in the matter.  They believe that granting the requests is in the public interest, and direct the Secretary to issue the order.

# O R D E R

- THE COMMISSION GRANTS the request of the City of Auburn to amend order TR-990655 to exclude the requirement to install a median barrier on the east side of the West Main Street highway-rail grade crossing, in Auburn, Washington.  It further grants the request of BNSF to modify the speed restriction to be a head-end restriction.

DATED at Olympia, Washington and effective this 26th of  July, 2002.

WASHINGTON UTILITIES AND TRANSPORTATION COMMISSION

CAROLE J. WASHBURN, Secretary