WN U-60	Original Sheet No. 85-l

# PUGET SOUND ENERGY

Electric Tariff G

# SCHEDULE 85

LINE EXTENSIONS

(Continued)

- POINT OF DELIVERY

  - For overhead or underground Primary voltage systems, the Point of Delivery and metering shall normally be at a point on the property line of the premises to be served that is, in the Company's judgment, most conveniently located with respect to the Company's transmission or distribution facilities.  The Company shall provide Primary metering facilities, and the incremental cost of such facilities in excess of Secondary metering facilities shall be included in the total costs to provide service under this schedule, and shall not be subject to offset against any Margin Allowance.  The Customer shall install, own, and maintain all facilities on the Customer's side of the Primary metering facilities.

  - For underground service at Secondary voltages to Non-Residential Customers, the Point of Delivery shall be at (i) the load side of the transformer or secondary handhole if located on the private property being served or (ii) the property line if the distribution facilities are located on a public right-of-way.  The transformer, handhole, or secondary connection location shall be at the point that is, in the Company's opinion, most conveniently located with respect to the Company's distribution facilities. The Customer shall install, own, and maintain all Secondary facilities beyond the transformer or Secondary handhole, except for metering equipment provided by the Company.

  - For underground service at Secondary voltages to Residential Customers, the Point of Delivery shall be at the Company side of the Customer’s meter base, except, when the Company provides, owns and maintains metering transformers, the Point of Delivery shall be at the Customer's side of such metering transformers.   Circuitry between the metering transformers and the Customer’s meter base and the meter shall be provided, installed and maintained by the Company.

  - For overhead service at Secondary voltages, the Point of Delivery  shall be at a point where the Company’s and the Customer’s circuitry interconnect on the outside of the structure to be served and that is, in the Company's judgment, most conveniently located with respect to the Company's distribution facilities.  The Customer shall install, own, and maintain all Secondary facilities beyond the Point of Delivery, except for metering equipment and metering circuitry provided by the Company.

(N)

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

|

(N)

Issued:  November 26, 2001								Effective:  December 27, 2001

Advice No. 2001-48

Issued By Puget Sound Energy

By:	    						     Director, Rates & Regulation

Steve Secrist