EXHIBIT NO. ___(CKC-1T)
DOCKET NO. UE-11___/UG-11___
2011 PSE GENERAL RATE CASE
WITNESS:  DR. CHUN K. CHANG

BEFORE THE

WASHINGTON UTILITIES AND TRANSPORTATION COMMISSION

| WASHINGTON UTILITIES AND TRANSPORTATION COMMISSION, Complainant, v. PUGET SOUND ENERGY, INC., Respondent. |  | Docket No. UE-11____ Docket No. UG-11____ |
| --- | --- | --- |

PREFILED DIRECT TESTIMONY (NONCONFIDENTIAL) OF

DR. CHUN K. CHANG
ON BEHALF OF PUGET SOUND ENERGY, INC.

JUNE 13, 2011

PUGET SOUND ENERGY, INC.

PREFILED DIRECT TESTIMONY (NONCONFIDENTIAL) OF
DR. CHUN K. CHANG

CONTENTS

I.	INTRODUCTION	1

II.	ELECTRIC AND GAS SALES WEATHER NORMALIZATION	2

A.	Normal Versus Actual Test Year Weather	3

B.	Temperature Adjustment of Electric Sales	4

C.	Temperature Adjustment of Gas Sales	11

III.	CONCLUSION	18

PUGET SOUND ENERGY, INC.

PREFILED DIRECT TESTIMONY (NONCONFIDENTIAL) OF
DR. CHUN K. CHANG

# I.	INTRODUCTION

Q.	Please state your name and business address.

A.	My name is Chun K. Chang, and my business address is 10885 N.E. Fourth Street, Bellevue, Washington 98004.  I am employed by Puget Sound Energy ("PSE") as a Regulatory Consultant in Pricing and Cost of Service.

Q.	Have you prepared an exhibit describing your education, relevant employment experience, and other professional qualifications?

A.	Yes.  It is the First Exhibit to my Prefiled Direct Testimony, Exhibit No. ___(CKC-2).

Q.	Please summarize the purpose of your testimony.

A.	The purpose of my testimony is to present PSE's electric and gas temperature adjustment methodologies and results used to develop the pro forma electric and gas sales for the test year in this proceeding, January through December 2010.

# II.	ELECTRIC AND GAS SALES WEATHER NORMALIZATION

Q.	Generally speaking, what is sales weather normalization and how does PSE perform its sales weather normalization?

A.	The sales weather normalization is performed to adjust the test year sales volume so that the adjusted sales represent what the test year sales volume would have been if the weather had been normal.  Weather normalization modifies the test year billing determinants and revenue requirements to be more representative of the average weather conditions expected when the rates proposed in this case go into effect.

PSE first analyzes the relationship between actual loads and temperatures for the most recent four-year period (2007 through 2010) and develops econometric models to measure temperature sensitivity of electric and gas energy use.  Multivariate regression analysis is used to isolate the weather effects from other factors such as type of day (e.g., weekdays, weekends or holidays) and seasonal effects not related to temperature.  The estimated model coefficients of temperature variables are called "weather sensitivity coefficients."

Then, PSE uses the weather sensitivity coefficients and "normal" weather data to convert the actual test year sales to "normal weather" sales.  PSE calculates the "normal" weather data from the actual historical temperature data compiled for the most recent thirty years.

Q.	Did PSE use the same weather normalization methodology in this case as in its last general rate case?

A.	Generally, yes; the methodology used in this case is the same temperature adjustment methodology PSE used in its 2009 general rate case ("2009 GRC"), except for a few minor improvements to be discussed later in this testimony.  The Commission approved the weather normalization analysis PSE performed in its 2009 GRC.

## A.	Normal Versus Actual Test Year Weather

Q.	Please describe the actual weather experienced during this proceeding's test year.

A.	Based on monthly history of heating degree days, Table 1 compares the actual monthly weather in the test year and the previous nine years with the normal weather defined by the average values calculated for the most recent thirty years of 1981-2010.  The hourly temperatures recorded at Seattle-Tacoma International Airport ("Sea-Tac") were used to calculate daily average temperatures.  The daily average temperatures were then converted to heating degree days ("HDDs") with a base temperature of 65˚F.  Monthly total HDDs were obtained by adding the daily HDD for each month.  For the test year, the overall weather, as measured by annual total HDD, was fairly close to normal.

Table 1

However, the deviation from normal weather was more substantial on a monthly basis, especially for mid-winter and mid-summer months.  As shown in the last column of Table 1, the winter weather in January, February and December was greater than 10 percent warmer than normal, while the weather in July and August was cooler than normal by 64.4 percent and 78.2 percent, respectively.

## B.	Temperature Adjustment of Electric Sales

Q.	Please describe how the electric sales temperature adjustment was calculated.

A.	The system-level temperature adjustment was calculated by month and allocated to each of the applicable rate schedules, based on a temperature adjustment methodology similar to the one used in PSE's 2009 GRC, with the hourly temperature and daily energy use data updated for 2009 and 2010.  The difference in methodology is described later in my testimony and is related to simplifying the mechanics of calculation.  Results of the simplified calculation would be the same as the complex calculation used in prior filings.

Q.	Please describe how the system-level test year load was normalized for weather.

A.	PSE used weather sensitivity coefficients based on actual daily load data and actual temperature data at Sea-Tac to adjust system-level delivered load (Generated, Purchased and Interchanged load, or "GPI") for weather.  The weather sensitivity coefficients were estimated by developing an econometric model with a four-year (2007-2010) history of daily GPI, HDDs and cooling degree days ("CDDs").  This is the same methodology PSE used in its last three general rate cases.

PSE's "normal" weather dataset was developed using the hourly temperature data recorded at Sea-Tac over the 30-year period from 1981 through 2010 by calculating daily HDDs and CDDs using several base temperatures (45˚F and 65˚F for HDDs; 60˚F and 65˚F for CDDs).  PSE then calculated the amount of temperature adjustment by taking the temperature variable coefficients from the econometric model and multiplying them by the difference between the actual and normal HDDs and CDDs.  This process was performed on a monthly basis and aggregated for all of the HDD and CDD variables included in the model.

Q.	How did you allocate the temperature adjustment among electric rate schedules?

A.	PSE used a three-step process to allocate the system-level temperature adjustment to rate schedules (classes) in order to produce rate schedule pro forma temperature-adjusted billing determinants.  The first step was to develop econometric model equations to characterize the relationship between the temperature variables and the daily energy use per customer by class.  The temperature variable coefficients of those equations vary by month and by rate class.  The data source for this step was a large sample of daily energy readings by rate schedule from PSE's automated meter reading database.  The historical data period set for modeling is the same four-year period of 2007 through 2010 as used for the system weather sensitivity modeling.

The second step was to calculate the temperature adjustment to monthly energy use per customer for each rate schedule by taking the temperature variable coefficients from the class model equation and multiplying them by the difference between the actual and normal HDDs and CDDs for the month.

The third step was to estimate monthly adjustment to class total sales by multiplying the monthly adjustment per customer calculated in the previous step by the actual number of customers by month and rate schedule.  The amount of monthly adjustment at the GPI level was allocated to each of the applicable schedules by calculating the percentage share of each schedule's adjustment amount relative to the sum of temperature adjustment for all classes as estimated through the rate class normalization process, and by multiplying the system total temperature adjustment by these percentage shares.

Q.	Has PSE made any change to the electric temperature adjustment method since its last general rate case?

A.	Yes.  PSE has simplified the second step in the procedure described above to estimate temperature adjustment to monthly energy use per customer by rate schedule.

Q.	What was the approach followed in the second step of estimating the rate class sales temperature adjustment in PSE's previous rate cases?

A.	In PSE's 2006, 2007 and 2009 GRCs, weather-normalized daily energy use by rate class was estimated by 1) simulating the daily energy use per customer using the actual daily average temperature history over the last thirty years and the rate-class model equations; and 2) taking the mean value of the simulation results for each of the corresponding weather days in the test year.  The temperature adjustment to per-customer energy use by class and month was then calculated by subtracting the actual energy use from the weather-normalized energy use.

Q.	How has PSE simplified the second step in this proceeding?

A.	Instead of going through the long process of simulating the daily energy use over thirty years, PSE calculated the temperature adjustment to monthly energy use per customer by class by applying the class temperature variable coefficients directly to the difference between monthly actual and normal HDDs and CDDs.

Q.	Does this simplified approach produce a different result?

A.	No.  Since the model coefficients of the weather variables are only differentiated by month (i.e., not by day), and because monthly HDDs and CDDs are simply the total daily HDDs and CDDs for the month, the simplified approach used in this case produces exactly the same results for the rate class sales adjustments as the previous approach, given the same model coefficients and thirty year temperature data.

Q.	Please summarize the final results of electric sales weather normalization.

A.	As shown in Table 2, below, applying the process described above to the test year GPI load of 22,594,274 megawatt hours ("MWhs") resulted in a total adjustment of 252,056 MWh, or 234,916 MWh delivered load when adjusted for losses.  Because the test year winter was warmer than normal, this adjustment resulted in a pro forma delivered system load that is larger than actual load delivered during the test year.  When the GPI temperature adjustment was allocated to the rate schedules, residential sales increased by 133,625 MWh and the loads of all but Schedule 29 Irrigation also increased.

Table 2

The irrigation load is sensitive only to the summer weather.  Monthly CDDs in August 2010 were 27.6 percent higher than normal while the weather in July 2010 was close to normal.  Consequently, the actual irrigation sales were lowered by 0.7 percent when the sales were temperature normalized.  Table 3 presents the temperature adjustment of electric sales by rate schedule.

Table 3

Q.	What is the effect of weather normalization on the electric revenue in the test year?

A.	The positive adjustment to electric load had the effect of increasing pro forma revenue by $20,896,426, as shown on page 6.01 of the Fifth Exhibit to the Prefiled Direct Testimony of John H. Story, Exhibit No. ___(JHS-6).

Q.	Is PSE's electric cost of service analysis and rate design study based on the weather-normalized sales?

A.	Yes.  Please see the Prefiled Direct Testimony of Jon A. Piliaris, Exhibit No. ___(JAP-1T), for an explanation of PSE's electric cost of service analysis and rate design study.  PSE's electric cost of service analysis includes the temperature-adjusted power costs, and the electric rate design is based on the pro forma adjustment of energy sales made for the milder than normal test year weather.  In addition, the energy cost allocation factors used in PSE's electric cost of service analysis reflect the temperature-adjusted loads.

## C.	Temperature Adjustment of Gas Sales

Q.	Please describe how the gas sales weather normalization was calculated.

A.	The system-level temperature adjustment was calculated in total and allocated to each of the applicable classes by month based on a gas temperature adjustment methodology similar to the one used in PSE's 2009 GRC and, most recently, in PSE's 2010 gas tariff increase proceeding, UG-101644.  The hourly temperature and daily and monthly gas sales data used for modeling were updated for this proceeding.

Q.	Please describe how the system-level gas throughput in the test year was normalized for weather.

A.	As was done in PSE's 2009 GRC and 2010 gas tariff increase proceeding, PSE used the weather-sensitivity model coefficients based on actual daily load data and actual temperature at Sea-Tac to adjust system-level delivered gas load (Firm, Interruptible and Transport) for weather.  The weather-sensitivity model coefficients were estimated on the basis of the daily gas load and weather data compiled for the most current four-year period of 2007 through 2010.  As with the electric model, PSE's "normal" weather dataset was developed using the hourly temperature data recorded at Sea-Tac over the 30-year period from 1981 through 2010.  Also consistent with the electric model, the actual daily HDDs were calculated using the average of the 24 hourly temperatures compared against the base temperature.  The amount of temperature adjustment was calculated by multiplying the weather sensitivity coefficients by the difference between the actual and normal HDDs. This calculation was performed on a monthly basis and aggregated for all of the HDD variables included in the system model.

Q.	How did you allocate the temperature adjustment among gas rate schedules?

A.	Initially, monthly gas usage patterns by rate schedule were evaluated to identify which rate classes are weather sensitive.  Monthly histories of class gas sales and HDDs were plotted for the most recent four years and the scattergrams were evaluated for any correlation between the changes in class gas sales and temperature.  This analysis revealed that the following rate classes are temperature sensitive:

- Schedule 23 (Residential),

- Schedule 31 (Commercial, Industrial),

- Schedule 41 (Commercial, Industrial),

- Schedules 85 (Commercial),

- Schedule 86 (Commercial),

- Schedule 87 (Commercial, Transport), and

- Special Contracts.

Econometric model equations were developed and estimated to characterize the relationship between monthly HDDs and average use per customer for each of the above weather sensitive classes.  For each month of the test year, the amount of temperature adjustment to system total delivered load was then allocated to each of the applicable classes by taking the percentage share of each schedule's temperature adjustment relative to the sum of temperature adjustment for all weather sensitive schedules as calculated by the class sales normalization equations, and by then multiplying the system load temperature adjustment by these percentage shares.

Q.	Has PSE made any change to the gas system temperature adjustment model since its 2009 GRC and 2010 gas tariff increase proceeding?

A.	Yes.  As shown in Table 1, the summer weather in July and August 2010 was unusually cooler than normal.  The 208 HDDs accumulated for those two mid-summer months were the highest in the last ten years.  Therefore, it appeared cool enough for some space-heating energy use, even in these summer months.  The firm load weather sensitivity model equation developed for this proceeding confirms this observation by showing statistical significance for the heating load in those two summer months.  Unlike in the 2009 GRC and 2010 gas tariff increase proceeding, the new gas model equation for the firm load therefore includes the HDD variable for July and August.  Consequently, the firm gas sales in July and August were adjusted slightly downward for normal summer weather.

Q.	Was there any change in the gas class models of temperature adjustment model?

A.	Yes.  PSE also evaluated the class-level gas sales for space-heating energy use in July and August.  The econometric modeling results of class gas sales reveal that some residential gas sales in those two mid-summer months were used for heating.  As in the case of the system model equation, but different from PSE's 2009 GRC and 2010 gas tariff increase proceeding, the residential class model equation includes the HDD variable for July and August.  Since the residential class is the only class of significant heating load in July and August 2010, all of the firm load adjustment made for those two months was allocated to the residential class.

Q.	Are the changes made to the system and rate schedule models in this case a departure from the theory and methodology approved in PSE's 2006 GRC?

A.	No.  These changes are merely updates and improvements to the existing methodology, reflecting the unusually cool summer weather experienced in 2010.

Q. 	Was there any unusual circumstance related to the historical gas sales data used for the class gas sales weather-sensitivity modeling?

A.	Yes.  Rate schedules 36, 51 and 57 were closed, and new rate schedules 41T, 85T and 87T were opened on November 1, 2008.  Since modeling requires a four-year history of class sales for January 2007 through December 2010, consistent monthly sales histories of the rate schedules had to be constructed for rate schedules 41T, 85T and 87T.  The monthly sales histories of those new rate schedules for January 2007 through October 2008 were built by tracking the monthly billings over this time period for the customers who were reclassified from the old rate schedules, 36, 51 and 57.

Q.	Please summarize the final results of gas sales weather normalization.

A.	Table 4 presents the temperature adjustment of sales by rate schedule.  As shown in the table, applying the process described above to the test year sales to the weather sensitive rate classes results in a total temperature adjustment of 32,926,082 therms.  Because the test year winter was warmer than normal, this adjustment resulted in a pro forma delivered system load larger than actual load delivered during the test year.  When the system temperature adjustment was allocated to the rate schedules, the gas sales to all of the weather-sensitive schedules were increased.  The residential class represented 64.7 percent of the total temperature adjustment, increasing by 21,293,690 therms.

Table 4

Q.	What is the effect of the temperature adjustment on revenue for the test year in this proceeding?

A.	The positive adjustment to volume had the effect of increasing pro forma revenue by $33,261,442 as shown on page 2 of the Second Exhibit to the Prefiled Direct Testimony of Janet K. Phelps, Exhibit No. ___(JKP-3).

Q.	Is PSE's gas cost of service analysis and rate design study based on the weather-normalized sales?

A.	Yes.  Please see the Prefiled Direct Testimony of Janet Phelps, Exhibit No. ___(JKP-1T), for a description of PSE's gas cost of service analysis and rate design study.  PSE's gas cost of service and rate design is based on the pro forma adjustment of energy sales made for the milder than normal test year weather.  In addition, the energy cost allocation factors used in PSE's cost of service analysis reflect the temperature-adjusted loads.

# III.	CONCLUSION

Q.	Does this conclude your testimony?

A.	Yes, it does.

See Docket No. UE-090704 and UG-009075 (consolidated), Order 11, ¶¶256-57 (April 2, 2010).

A heating degree day (HDD) is the negative deviation in average daily temperature from the base temperature.  For a base temperature of 65˚F, heating degree days equal 65 minus the average daily temperature (if the average temperature is less than 65).  If the average daily temperature is greater than 65, HDD is 0.  Thus, one day that averages 35˚F would have 30 HDDs (using a base of 65˚F).  Similarly, 30 days with an average temperature of 64˚F each day would also have 30 HDDs.

A Cooling Degree Day is calculated in the same way as a Heating Degree Day is calculated, except that it counts number of degrees above the base temperature.