## ELECTRIC COST OF SERVICE

A cost of service study is an engineering-economic study, which apportions the revenue, expenses, and rate base associated with providing electric service to designated groups of customers.  It indicates whether the revenue provided by the customers recovers the cost to serve those customers.  The study results are used as a guide in determining the appropriate rate spread among the groups of customers.

There are three basic steps involved in a cost of service study: functionalization, classification, and allocation. See flow chart.

First, the expenses and rate base associated with the electric system under study are assigned to functional categories.  The uniform system of accounts provides the basic segregation into production, transmission, and distribution.  Traditionally, customer accounting, customer information, and sales expenses are included in the distribution function and administrative and general expenses and general plant rate base are allocated to all functions.  In this study I have created a separate functional category for common costs.  Administrative and general costs that cannot be directly assigned to the other functions have been placed in this category.

Second, the expenses and rate base items which cannot be directly assigned to customer groups are classified into three primary cost components:  energy, demand or customer related.  Energy related costs are allocated based on each rate schedule’s share of commodity consumption.  Demand (capacity) related costs are allocated to rate schedules on the basis of each schedule’s contribution to peak demand.  Customer related items are allocated to rate schedules based on the number of customers within each schedule.  The number of customers may be weighted by appropriate factors such as relative cost of metering equipment.  In addition to these three cost components, any revenue related expense is allocated based on the proportion of revenues by rate schedule.

The final step is allocation of the costs to the various rate schedules utilizing the allocation factors selected for each specific cost item.  These factors are derived from usage and customer information associated with the test period results of operations.

BASE CASE COST OF SERVICE STUDY

Production and Transmission Classification (Peak Credit)

This study utilizes a Peak Credit methodology to classify production and transmission costs into demand and energy classifications.  The Peak Credit method acknowledges that baseload production facilities provide energy throughout the year as well as capacity during system peaks and likewise the transmission system is built not only for peak use, but also for everyday delivery of energy.  The demand/energy ratio is determined by the relationship of the current replacement cost per kW generating capacity of the Company’s peaking units to the current replacement cost per kW generating capacity of the Company’s thermal or hydro plant.  The peak credit ratio for thermal plant is 33.57% to demand and 66.43% to energy.  The peak credit ratio for hydro plant is 26.82% to demand and 73.18% to energy.  As an intermediate resource (between peaking and baseload) Coyote Springs II has been included with the thermal plant costs, while Boulder Park and Kettle Falls CT have been included with the peaking units.

Transmission costs are classified by fifty-fifty weighting of the thermal and hydro peak credit ratios resulting in the transmission peak credit ratio of 30.19% to demand and 69.81% to energy.  Fuel and load dispatching expenses are classified entirely to energy.  Peaking plant related costs are classified entirely to demand.  Purchased Power and Other Power Supply expenses are classified to demand and energy by the relative amounts of assigned and allocated Production Plant in Service.  WNP3 Settlement Exchange Power costs are classified as energy related.

Production and Transmission Allocation

Production and transmission demand related costs are allocated to the customer classes by class contribution to the average of the twelve monthly system coincident peak loads.  Although the Company is usually technically a winter peaking utility, it experiences high summer peaks and careful management of capacity requirements is required throughout the year.  The use of the average of twelve monthly peaks recognizes that customer capacity needs are not limited to the heating season.

Energy related costs are allocated to class by pro forma annual kilowatthour sales adjusted for losses to reflect generation level consumption.

Distribution Facilities Classification (Basic Customer)

The Basic Customer method considers only services and meters and directly assigned Street Lighting apparatus (FERC Accounts 369, 370, and 373 respectively) to be customer related distribution plant.  All other distribution plant is then considered demand related.  This division delineates plant which benefits an individual customer from plant which is part of the system.  The basic customer method provides a reasonable, clearly definable division between plant that provides service only to individual customers from plant that is part of the interconnected distribution network.  Additionally, the basic customer method has been explicitly accepted for both electric and gas cost of service in the State of Washington.

Customer Relations Distribution Cost Classification

Customer service, customer information and sales expenses are the core of the customer relations functional unit which is included with the distribution cost category.  For the most part they are classified as customer related.  Exceptions are sales expenses which are classified as energy related and uncollectible accounts expense which is considered separately as a revenue conversion item.

Distribution Cost Allocation

Distribution demand related costs which cannot be directly assigned are allocated to customer class by the average of the twelve monthly non-coincident peaks for each class.  Distribution facilities that serve only secondary voltage customers are allocated by non-coincident peak excluding all primary and transmission voltage customers.  This includes line transformers, services, and secondary voltage overhead or underground conductors and devices.  Primary voltage overhead or underground conductors and devices are allocated by non-coincident peak for all customers except the one transmission voltage customer.

Most customer costs are allocated by average number of customers.  Weighted customer allocators have been developed using typical current cost of meters, estimated meter reading time, and direct assignment of billing costs for hand-billed customers.  Street and area light customers are excluded from metering and meter reading expenses as their service is not metered.

Administrative and General Costs

Administrative and general costs which are directly associated with production, transmission, distribution, or customer relations functions are directly assigned to those functions and allocated to customer class by the relevant plant or number of customers.  The remaining administrative and general costs are considered common costs, and have been left in their own functional category.  These common costs are allocated to rate class by factors equivalent to those approved for Puget Sound Power and Light (now PSE) in Docket No. UE-920499 and indirectly classified by the implicit relationship of energy, demand and customer that make up the various allocation factors applied to the costs.

Common plant items are allocated to rate class by either relative: production, transmission, distribution plant; production, transmission, distribution labor subtotal; or operating and maintenance labor total.  Most common administrative and general expenses are allocated to rate class by relative operating and maintenance expenses before administrative and general expenses excluding purchased power, fuel, wheeling, and revenue items.  Property insurance expense is allocated by plant totals.  Injuries & damages and pensions & benefits expenses are allocated by operating and maintenance labor expense totals.

Revenue Conversion Items

In this study state excise tax, uncollectible accounts and commission fees have been classified as revenue related and are allocated by pro forma revenue.  These items vary with revenue and are included in the calculation of the revenue conversion factor.  Income tax expense items are allocated to schedules by net income before income tax adjusted by interest expense.

For the functional summaries on pages 2 and 3 of the cost of service study, these items are then assigned to component cost categories.  The revenue related expense items have been reduced to a percent of all other costs and loaded onto each cost category by that ratio.  Similarly, income tax items have been reduced to a percent of net income before tax then assigned to cost categories by relative rate base (as is net income).

The following matrix outlines the methodology applied in the Company Base Case cost of service study.