| BEFORE THE WASHINGTON STATE UTILTIES AND TRANSPORTATION COMMISSION |  |
| --- | --- |
| In the Matter of the Petition for Arbitration of an Interconnection Agreement Between CHARTER FIBERLINK WA-CCVII, LLC. and QWEST CORPORATION Pursuant to 47 U.S.C. Section 252 (b) | DOCKET NO. UT-083041 |

REBUTTAL TESTIMONY OF Philip Linse

QWEST CORPORATION

(Disputed Issue Nos. 5, 10, 11, 13, 15, and 16)

NOVEMBER 17, 2008

TABLE OF CONTENTS

I.	IDENTIFICATION OF WITNESS	1

II.	PURPOSE OF TESTIMONY	1

III.	DISPUTED ISSUE NO.  5: LIMITATION OF LIABILITY	2

IV.	DISPUTED ISSUE NO. 10:  INTERCONNECTION FACILITY OPTIONS	3

V.	DISPUTED ISSUE NO. 11:  METHODS OF INTERCONNECTION	4

VI.	DISPUTED ISSUE NO. 13: TRANSPORT OBLIGATIONS	8

VII.	DISPUTED ISSUE NO. 15: BILL AND KEEP COMPENSATION	9

VIII.	DISPUTED ISSUE NO. 16: INDIRECT INTERCONNECTION	10

IX.	CONCLUSION	14

# I.	IDENTIFICATION OF WITNESS

Q.	PLEASE STATE YOUR NAME, OCCUPATION AND BUSINESS ADDRESS.

A.	My name is Philip Linse.  My business address is the Qwest Network Reliability Center located at 700 W. Mineral Avenue, Littleton Colorado.  I am employed as Director – Public Policy for Network.  I am testifying on behalf of Qwest Corporation (“Qwest”).

Q.	ARE YOU THE SAME PHILIP LINSE WHO FILED DIRECT TESTIMONY IN THIS DOCKET?

A.	Yes.

# II.	PURPOSE OF TESTIMONY

q.	what is the purpose of your testimony?

A.	The purpose of my testimony is to respond to the Charter Fiberlink WA-CCVII, LLC (“Charter”) testimonies of Mr. Starkey and Mr. Gates.  I first respond to portions of Mr. Starkey’s testimony regarding Issue 5: Limitation of Liability.  Next, I reply to Mr. Gates’ testimony as it relates to the following disputed issues:

    - Issue 10:  Interconnection Facility Options

    - Issue 11:  Methods of Interconnection

    - Issue 13:  Transport Obligations

    - Issue 15:  Bill and Keep Compensation

    - Issue 16:  Indirect Interconnection

# III.	DISPUTED ISSUE NO.  5: LIMITATION OF LIABILITY

Q.	IS IT LIKELY THAT A QWEST TECHNICIAN WOULD DAMAGE CHARTER’S FIBER THROUGH THE ACT OF INTERCONNECTING QWEST’S NETWORK WITH CHARTER’S NETWORK AS MR. STARKEY CLAIMS ON PAGE 8 OF HIS DIRECT TESTIMONY?

A.	No.  Interconnection with Charter may take place at a Charter collocation within Qwest’s central office, at a Mid-Span Meet POI where Qwest’s fiber facility is met with Charter’s fiber facility, or a Charter premise location where Qwest provides entrance facilities for Charter.  To the extent Qwest provides entrance facilities to Charter’s premise or interconnects with Charter at a collocation, Qwest and Charter would connect their networks using termination blocks and cross connects.  At the time interconnection takes place, both Qwest’s fiber facilities and Charter’s fiber facilities should be properly terminated with the appropriate grounding.  The method of connecting two networks using termination blocks and cross connects dramatically reduces the possibility of damage to the facilities used for interconnection.  If Qwest and Charter were to use a fiber Mid-Span Meet POI, Qwest’s fiber and Charter’s fiber should be spliced together using fiber facilities used solely for the purpose of interconnection.  This reduces potential damage to either party’s network.  At the time interconnection takes place both Qwest fiber facilities and Charter fiber facilities should be presented for splicing in a way that properly protects each party’s network.  Thus, it is very unlikely that a Qwest technician would damage Charter’s fiber through the act of interconnecting Qwest’s network with Charter’s network.

# IV.	DISPUTED ISSUE NO. 10:  INTERCONNECTION FACILITY OPTIONS

q.	On page 13 of his testimony Mr. Gates states that “the parties have agreed that when a tandem switch nears exhaust, there may be “technical or operational CONCERNS” that Would render INTERCONNECTION at that switch TECHNICALLY infeasible.  If the parties agree that switch EXHAUST would render interconnection technically INFEASIBLE, should there be a requirement That switch exhaust be demonstrated to the COMMISSION in every instance?

A.	No.  It is clear by Mr. Gates’ statement that Charter understands the nature of switch exhaust and the technical limitations that exist when a switch is in a near exhaust condition.  There is very little to disagree about when a switch has reached its capacity.  As Qwest’s and other CLECs’ experience have improved the accuracy of network forecasting, Qwest has been able to work with interconnecting carriers to limit the impact of switch exhaust conditions.  Generally, CLECs have been cooperative and understanding of these situations and have been helpful in assisting Qwest in its efforts to minimize the impact of switch exhaust.  Thus, it is administratively unnecessary and potentially burdensome for the Commission to become involved in evaluating whether there is switch exhaust every time it happens.

# V.	DISPUTED ISSUE NO. 11:  METHODS OF INTERCONNECTION

Q.	Mr. Gates claims that Qwest’s proposed language limits the methods by which Charter may establish interconnection.  Is he correct?

A.	No.  As my direct testimony explains, there are several standard methods of interconnection that Qwest provides.  Charter may obtain interconnection through the use of a Qwest provided Entrance Facility where Qwest builds a transmission path between Qwest’s serving central office building and Charter’s location.  Charter may also obtain its own transport to a collocation within Qwest’s central office.  Finally, Charter may choose to negotiate a Mid-Span Meet POI where Charter provides or obtains facilities to a negotiated point approximately half way between the Charter point of presence within Qwest’s local service area and the Qwest serving central office building.  In addition, if these three methods do not meet Charter’s needs, then Charter may elect to request an alternate method through the Bona Fide Request (“BFR”) process.  The BFR process allows for CLECs such as Charter to request technically feasible alternate methods of interconnection that are not covered by the three I have discussed.

Q.	Has Charter submitted a BFR for a method of interconnection that is not covered by the three methods that you have described?

A.	No.  I am not aware of any request from Charter for a method of interconnection that is not covered by the three methods of interconnection that I have described.

Q.	is a Qwest provided entrance facility designed for interconnection with multiple service providers?

A.	No.  A Qwest provided Entrance Facility involves circuits that are dedicated for interconnection between the requesting service provider’s switch and Qwest’s switch.  These circuits are configured using the software of the switch.  A trunk is the software configuration of a circuit between two switches.  A trunk has the capability of establishing one voice grade communications path between two switches.  A trunk group may consist of multiple trunks.  Thus, a trunk group may carry many trunks that provide many voice paths between two switches.  It is technically impossible to operate the same trunk or trunk groups between more than two switches.  This would be analogous to providing the same mailing address for two different households.  The mail carrier would not know where to deliver the mail.  Likewise, Qwest’s switch would not know which CLEC switch to route calls to.

Q.	Does a Qwest provided entrance facility restrict or limit Charter from obtaining transport facilities from a third party that may be used for interconnection as Mr. Gates concludes on page 19 lines 7 through 9 of his direct testimony?

A.	No.  Charter may obtain the equivalent of Qwest provided Entrance Facilities through the use of another provider.  This may be accomplished through the use of its own collocation or that of another service provider’s collocation.  Charter may obtain transport from a third party between its collocation located at a Qwest central office and Charter’s network.  Additionally, Charter may build these facilities itself.  Finally, Charter may use a Mid-Span Meet POI and obtain its portion of the facilities from a third party.  Thus, on page 19 lines 7 through 9, Mr. Gates ignores the options available to Charter and the fact that neither Qwest nor Qwest’s language restricts Charter from using a third party’s facilities as a way for Charter to interconnect  with Qwest.

In fact, Qwest proposed language for section 7.2.2.1.2.2 allows a CLEC to purchase transport services from a third party, including a third party that has leased the private line transport service facility from Qwest.

Q.	does Qwest become involved in charter’s or other CLEC’s lease of third party transport facilities that may be used to facilitate interconnection?

A.	No.  Qwest is not a party to the agreements between CLECs such as Charter and third parties.  Although Qwest is not involved, Qwest may need information in order to arrange for interconnection between Qwest and Charter using a third party’s facilities.  Qwest would need to know the location of the third party’s facility and authorization to use the facility for interconnection between Qwest and Charter.

Q.	to what extent do CLECs use facilities of other service providers for interconnection with Qwest?

A.	In the state of Washington at least 7 service providers use the facilities of at least 8 other service providers to interconnect with Qwest.  Thus, Mr. Gates’ claim that Qwest’s proposed language prohibits Charter from using other service providers’ facilities is wrong.  It is clear that Qwest’s proposed language allows for the use of third party facilities for interconnection with Qwest.

Q.	Does Charter use the facilities of a third party service provider in Washington?

A.	Yes.  Charter interconnects with Qwest through the use of a third party.  Confidential exhibit PL-8 is a letter of authorization provided to Qwest by a third party which allows Charter to use that third party’s facilities to interconnect Charter’s network with Qwest’s network.

Q.	On page 25 of his direct testimony Mr. Gates claims that Qwest may force CLECs to duplicate Qwest’s network.  Does Qwest’s proposed language require CLECs such as Charter to duplicate Qwest’s network?

A.	No.  Qwest’s language specifically allows one POI per LATA if it is technically feasible.  The duplication of Qwest’s network is not a requirement of Qwest’s proposed language nor does Qwest force CLECs like Charter to duplicate Qwest’s network as Mr. Gates suggests.

Q.	Does Qwest UNILATERALLY Require Charter to establish multiple POIs as Mr. Gates claims on page 26 of his direct testimony?

A.	No.  Qwest has no such requirement. In fact, the second sentence of Qwest’s proposed language for Section 7.1.2 states:

CLEC shall establish at least one (1) physical Point of Interconnection in Qwest territory in each LATA CLEC has local End User Customers.

Q.	Does Qwest require multiple POIs as Mr. Gates claims on page 25 of his direct TESTIMONY?

A.	No. As I have explained above, Qwest offers interconnection to Charter from a single location within Qwest’s service territory and has developed multiple interconnection methods that may be used.

# VI.	DISPUTED ISSUE NO. 13: TRANSPORT OBLIGATIONS

Q.	On page 17 and again on page 29 of his direct testimony Mr. Gates claims that “Charter’s proposal makes clear that Qwest has no obligation to establish a POI with Charter outside of Qwest’s GEOGRAPHIC TERRITORY or service area.”  Does Charter’s language consistently reflect this position?

A.	No.  Charter’s language at 7.1.2 that states ”The parties agree that this Section 7.1.2 shall not be construed as imposing any obligation upon Qwest to establish a physical point of interconnection with Charter that is outside of Qwest’s geographic service area or territory.”  By its terms, this language is limited to Section 7.1.2 of the interconnection agreement and does not expressly apply to other sections of the interconnection agreement.  Mr. Gates states “that Qwest has no obligation to establish a POI with Charter outside of Qwest’s geographic territory or service area” but his statement is not reflected in all of the contract language that Charter actually proposes.  For example, as I discussed in my direct testimony, Charter’s proposed language for section 7.2.2.1.2.2 could be interpreted to require Qwest to provide transport for interconnection outside of Qwest’s service territory.

# VII.   DISPUTED ISSUE NO. 15: BILL AND KEEP COMPENSATION

Q.	Does Charter’s bill and keep proposal for transport, as Mr. Gates EXPLAINS on page 37 of his direct testimony, adequately compensate qwest for transport qwest provides?

A.	No.  Mr. Gates makes an unfounded and totally incorrect assumption that “since the parties expect the volume of traffic will be roughly balanced, the parties’ respective costs of transporting and terminating the other party’s traffic should be roughly balanced.”  Qwest makes transport available to Charter between Charter’s POI to over 45 central office switches.  Charter provides transport from its POI to its single switch location.  Clearly, Qwest provides substantially more transport than Charter provides and this is true notwithstanding that the amount of traffic flowing in each direction may be in rough balance.  Exhibit PL-9 is an example that illustrates how much more transport is provided by Qwest than is provided by Charter.

# VIII.	DISPUTED ISSUE NO. 16: INDIRECT INTERCONNECTION

Q.	On page 53 of his direct testimony Mr. Gates claims that Charter’s proposed language allows the parties to exchange local traffic indirectly.  Does Charter’s language provide Qwest with a choice of INDIRECT interconnection with Charter?

A.	No.  Charter’s proposed language for section 7.1.2.7 requires Qwest to route traffic to Charter indirectly.  Qwest should not be forced into an indirect connection if Qwest determines direct interconnection is the best option for Qwest’s business.

Q.	On page 53 and 54, mr. gates claims that 240,000 MINUTES FOR three consecutive months is a REASONABLE threshold before direct interconnection may be allowed.  Is the threshold proposed by Charter based upon any telecommunications standard of which you are aware?

A.	No.  As I explained in my direct testimony Charter’s proposed benchmark does not appear in any telecommunications industry standard of which I am aware.

Q.	does Charter’s proposed threshold provide a usable method to determine when direct interconnection should be implemented?

A.	No.  Charter’s proposed threshold appears to be an arbitrary number that Charter has developed.  It does not appear to be based upon any telecommunications traffic engineering theory or industry practice for traffic engineering.

Q.	What problem does using a threshold as proposed by Charter present?

A.	There appears to be no engineering calculation that can specifically determine what network capacity would be required to accommodate Charter’s proposed threshold.  Charter’s proposal does not account for telecommunications traffic patterns.  As a result, Qwest would have little idea of what network capacity it would be required to have in place or if Qwest would be better served using direct trunking with Charter.  For example, as an extreme illustration, imagine that Charter has 240,000 customers and they all called Qwest’s customers at the same time for one minute.  Charter would have reached its benchmark in one minute of its three month period.  Additionally, this would require 240,000 voice grade circuits between Qwest and the third party transit provider.  Similarly if only 2 people used their phone for 24 hours a day for 3 months the 240,000 minute threshold would be met near the 83rd day of the 3 month (90day) timeframe.  This would only require 2 voice grade circuits between Qwest and a third party transit provider.  Charter’s benchmark may justify direct connections in the first example but may not justify direct connection in the second example.  Under Charter’s proposed threshold, Qwest has no way to determine if Charter’s proposal is reasonable or whether it could potentially risk Qwest’s network reliability.

Q.	Does Mr. Gates or Charter’s proposed language explain how the parties would select A THIRD PARTY TRANSIT PROVIDER for indirect interconnection?

A.	No.  Neither Mr. Gates nor Charter’s proposed language explain how Qwest would know what other service providers Charter has connections with.  Although the Local Exchange Routing Guide (“LERG”), a telecommunications industry wide routing guide, shows that Qwest has many service providers that are interconnected with Qwest’s network, the LERG does not indicate the service providers connected to Charter.  In fact, the LERG currently does not show any option for Qwest to indirectly interconnect with Charter.  However, even if the LERG did show a service provider with connections to Charter, Qwest would still not know if the provider was the provider Charter would use to exchange traffic with Qwest.

Q.	Does this also create concerns to Qwest?

A.	Yes.  Although Qwest has many interconnected service providers, Qwest may only have limited options to route traffic to Charter indirectly.    Consequently, Charter’s proposed language may subject Qwest to undesirable terms and conditions with the available third party transit providers.  Conversely, for traffic originated by Charter, Charter’s language may allow Charter to route traffic to Qwest using indirect connections of which Qwest is not aware and could lead to unidentifiable traffic.  Thus, as I also explained in my direct testimony, Charter’s proposal would create opportunities for the creation of phantom traffic.  Charter’s proposed language does not address any of these potential problems.

Q.	On page 54 of his direct testimony Mr. Gates’ explains that “once a direct interconnection is established between the parties, indirect interconnection may no longer be used except in overflow conditions.”  Does Mr. Gates or Charter’s proposed language define overflow or how traffic associated with indirect interconnection would terminate to Qwest’s network?

A.	No. Qwest’s network relies upon tandem switches and end office switches for terminating traffic to Qwest end users.  Neither Mr. Gates nor Charter’s proposed language describes how indirect traffic is expected to route into Qwest’s network.  Charter’s proposal is not clear as to whether Charter’s originated traffic would route indirectly to Qwest at Qwest’s tandem switches or indirectly to Qwest’s end office switches.

Q.	Why is it important to agree how this traffic will be routed between the parties?

A.	It is important because of the impact to customers.  As additional switches are included within the call path, service quality can be impacted.  When more that two switches are involved, it is necessary to adjust the engineering of the circuits so that customer impact is minimized.  Otherwise customers may experience higher than normal call blocking.  Charter’s proposal does not take into account the customer service impacts it could have.

# IX.	CONCLUSION

Q.	DOES THIS CONCLUDE YOUR TESTIMONY?

A.	Yes.