Wind Integration Study were modeled in GRID using the same methodology employed by the Company to model reserves for load and wind in its filing.

REVISED 12/29/2014

Q.	WHAT AMOUNT OF WCA RESERVE SAVINGS DID THE 2012 WIND INTEGRATION STUDY ASSOCIATE WITH 30-MINUTE BALANCING?

A.	The 2012 Wind Integration Study calculated that the WCA regulation reserve requirement would decline by approximately 30 percent as a result of moving to 30-minute balancing.76/  Moving to 5-minute balancing, as is accomplished in the EIM, will likely result in an even greater level of reserve savings.

Q.	DO THE WITHIN-HOUR DISPATCH BENEFITS OVERLAP WITH FLEXIBILITY RESERVE DIVERSITY?

A.	No.  The E3 study was clear when it stated: “Production simulation analysis [was] modeled at [an] hourly level, omitting potential benefits of sub-hourly dispatch (other studies indicate that these benefits could be substantial).”77/  In addition, because the various EIM benefit components have been modeled in GRID, the final balancing adjustment detailed in Table 2 removes any overlaps between components.

Q.	WHAT IS THE IMPACT OF MODELING THE RESERVE REDUCTIONS ATTRIBUTABLE TO 30-MINUTE BALANCING PRESENTED IN THE 2012 WIND INTEGRATION STUDY?

A.	Modeling the approximate 30 percent reduction to regulation reserves in the GRID model study resulted in a $3.0 million reduction to WCA NPC, with $702,450 allocated to Washington.  This amount represents a conservative provision for the savings associated with within-hour EIM dispatch benefits.

7	6/ 	Id. at 123.

7	7/ 	Exh. No.___(BGM-5 at 37)