911 Facility Based Process Task List

| Assoc. Task # | Process |
| --- | --- |
| 1 | CLEC must have DS1 in place via ASR process for 911 circuits. |
| 2 and 3 | Obtain preorder/order information and network requirements. Account team completes RG2900929. |
| 4 | Network (O&T) verifies that facilities are available to 911 router location(s). |
| 5 | If no trunking in place, CLEC must issue ASR. |
| 6 | Service order issued by 911 Center. Word Doc issued for facilities and circuit paths. |
| 7 | CLEC interval validated based on network critical dates. |
| 8 | Account team provides final due date and refers CLEC to SCC for Database administration. Service order issued for data link between the CLEC and SCC for 911 record update transmissions to ALI database. |
| 9 | Account team confirms authorization to proceed with 911 Center. |
| 10 | Service order is issued into Service Order Processor (SOP). Final network designs issued. |
| 11 | Firm Order Commitment (FOC) is issued to CLEC. Trunk design, facility assignments, and translations are determined. |
| 12 & 13 | Complete Word Doc and final translations are made. |
| 14 | Network connections in switch and facilities are made. |
| 15 | Verification of work is performed to determine if ready for testing. |
| 16 & 17 | 911 Center coordinates testing with the CLEC and PSAP to ensure 911 calls can be made according to standards. |
| 18 | CLEC approves work is complete and accepted. Orders are completed in all systems. |

911 Resale Task List

| Assoc. Task # | Process |
| --- | --- |
| 1 | Local Service Request (LSR) form submitted to Denver Interconnect Service Center (ICS). |
| 2 and 3 | Obtain preorder/order information and review LSR and associated forms for completeness. |
| 4 | Service Order is issued into Service Order Processor (SOP). |
| 5 | Service is provisioned and order is completed in SOP. |
| 6 | Transaction Services System (TSS) electronically searches each SOP for completed orders and batch uploads between 7 & 10 PM each business day. |
| 7 | TSS electronically reviews customer record for any 911 relevant data fields; check format and matches address information against the MSAG for accuracy. If OK record is entered by TSS into ALI – DB. If not, record errors out for analyst to resolve discrepancy. |
| 8 | When record loaded in ALI, SCC maintains the record for future use should a 911 call be made. At the time of a 911 call, a query from the selective router causes the ALI records associated with a particular telephone number to be sent to the caller’s PSAP. |

911 Unbundled Network Elements Task List

| Assoc. Task # | Process |
| --- | --- |
| 1 | Initiate the Unbundled Network element Process. |
| 2 | Complete a service order (service request)to Migrate. |
| 3 | Sends SCC subscriber data to Migrate. |
| 4 | Complete Service Order to "Disconnect" service. |
| 5 | Sends SCC subscriber data to "Unlock" 911 record. |
| 6 | Attempts to Unlock the existing subscriber data. |
| 7 | Evaluate subscriber data sent from Co-Provider. |
| 8 | Resolve issues associated with SC ability to unlock record. |
| 9 | Notify Co-Provider of errors associated with subscriber data to Migrate existing records. |
| 10 | Resolve errors and resubmit to SCC. |
| 11 | Migrate records. |

911 Facility Based Repair Task List

| Assoc. Task # | Process |
| --- | --- |
| 1 & 2 | Trouble ticket submitted. CLEC must report trouble to the 911 Care Center(1-800-357-0911). With the trouble, state the company they represent and provide a circuit ID #. (This is the same procedure required by U S WEST for its retail (PSAP) customers.) |
| 3 | Process ticket received from CLEC. Open trouble ticket in Work Force Administration system (WFA-C) and enter error codes. |
| 4 | Trouble ticket created. Begin system tests to isolate through circuit test points if available. |
| 5 | Analyze trouble ticket, identify location, and assign to appropriate organization for repair. |
| 6 | When trouble location cannot be identified, ticket assigned to Installation & Maintenance Technician. This may require a dispatch of personnel to Central Office ( Switch) locations or to the field. |
| 7 | Trouble ticket received in Network Operations. Trouble is isolated. Trouble repaired. Trouble ticket updated for status to center and customer, or closed out in the system with results of problem and corrective action or work performed noted. Customer is notified to ensure acceptability and verify that service is restored. |
| 8 | 911 Center closes out trouble ticket in the system and final acceptance by customer is verified. |