Docket No. UT-023003

Verizon Responses to Commission Bench Requests

June 21, 2004

Bench Request No. 4:

For Exhibit 283, page 7, provide updated page with two years actual and 3 years projected information.

Response:

The updated inflation and productivity factors are shown on the attached file, “ATT_4_4,5 update April 2004.xls.”  This file includes the actual index data for 2002 and 2003 through the first quarter of 2004 and projected data for 2004 through 2006 from economy.com, an independent consulting firm.

Docket No. UT-023003

Verizon Response to Commission Bench Request

June 21, 2004

Bench Request No. 7

A)	Provide the switching cost study rolled up total investment and totals lines.

Response:

The total switch investment and total lines can be found in Verizon NW’s switching study support files, on CD #2 filed with the Panel Direct Testimony on June 26, 2003. Details of the specific data requested are located in the attached Excel file “Total Investment and Lines.xls”.

However, simply dividing these two numbers will not give an accurate depiction of the costs associated with a flat-rated POTS port.  The switch investment alone does not recover all of the investments involved in local switching.  Also, simply dividing total switch investment by total switch lines provides an average investment per port across all types of ports, but this ignores investment differences between POTS, ISDN BRI and ISDN PRI ports and would inflate the monthly cost of a POTS port by averaging it with the more expensive ISDN ports.

Although Verizon NW does not believe that a flat UNE switching rate would satisfy cost causation principles, it provides the following instructions for calculating a flat rate:

Begin with the results for Local End Office Switching.  Per Verizon Exhibit RP-2, dated June 26, 2003, the rates for Local End Office Switching are:

Originating EO Local Switching		$0.004978

Terminating EO Local Switching		$0.004423

An example of the following steps, as calculated using the cost studies filed by Verizon on June 26, 2003, is provided in the Excel file embedded below (double-click on the icon to activate Excel and open workbook).

Steps to create flat-rate usage:

- Develop the average EO Local Switching rate per minute of use, simply the sum of the originating + terminating divided by 2.

- Extract the total annual minutes of use from the Jurisdictional Demand BC family.  The total annual minutes value is the number associated with the item “Annual_Line_MOU.”

- Multiply the average EO Local Switching rate per minute of use by total annual minutes to derive the total annual EO Local Switching cost.  Divide that result by 12 to calculate the monthly EO Local Switch cost total.

- Extract from the Jurisdictional Demand BC family the values for total POTS lines, total BRI lines and total PRIs.  These are the items named “POTS_Lines_Total,” “BRI_Lines_Total” and “PRI_Lines_Total.”  Multiply the total PRIs by 23 to develop PRI DS0 equivalents.

- Extract the utilization factors for POTS, BRI and PRI from the file named WA 5ESS Model.xls (provided with the filing on CD#2 – UNE Proprietary CD\5. Cost Study Support Files\Switching & Common Transport\Switch Cost Study Support Files).  Although the file contains the 5ESS investments, the utilizations are the same for all switch types.  This may be verified by viewing the same tab in the DMS and GTD5 model workbooks.

- Divide the POTS, BRI and PRI (DS0 equivalents) lines by their respective utilizations and sum them to determine the total working DS0s in the network

- Divide the monthly EO Local Switching cost (from Step 3) by the total DS0s (from Step 6) to calculate the total EO Local Switching cost per month per line.

- To calculate the total POTS flat-rated port cost per month per line add the EO Local Switching cost per month per line from the previous step to the rate for the POTS/PBX/CTX Port.

- To calculate the total ISDN BRI flat-rated port cost per month per line add the EO Local Switching cost per month per line from Step 7 to the rate for the ISDN BRI Port.

- To calculate the total ISDN PRI flat-rated port cost per month per PRI add the EO Local Switching cost per month per DS0 from Step 7, times 23, to the rate for the ISDN PRI Port.

B) 	Does the ARMIS switched line count include UNE-P lines?

Response:

The ARMIS line count discussed at the hearings was from the column (cj) in the ARMIS 43-08 report (Total Switched Access Lines). This is a facilities-based report and therefore reflects all lines regardless of what type of service they provide, including UNE-P lines.

Docket No. UT-023003

Verizon Responses to Commission Bench Requests

June 21, 2004

Bench Request No. 14

Verizon HAI Panel:  Supply Mr. Dippon’s workpapers supporting his comparison of strand distance v. road distance.

Response:

The workpapers for Mr. Dippon’s study are attached in the zip-file labeled “Record_Request.zip.”  Mr. Dippon used MapInfo 7.0 for his analysis and conducted a series of steps to obtain the road miles for each cluster in the Richmond Beach wire center.

First, Mr. Dippon created a table of cluster road segments in MapInfo by performing the following SQL queries:

Select Hulls.Cluster, WCRds.SegID

From WCRds, Hulls

Where WCRds.Obj Intersects Hulls.Obj

Commit Table Selection As “WCRds_Cluster”

Mr. Dippon then saved this table as “WCRds_Cluster.tab.”  Second, Mr. Dippon opened the “Hulls” and “WCRds_Cluster” tables in a Map window within MapInfo.  He then made the WCRds_Cluster layer editable and performed the following steps for each cluster:

- Select the road segments for the wire center through the following SQL query:

Select * From WCRds_Cluster Where Cluster = n (where n = cluster number)

- Activate the Map window and set the selected roads for the cluster as the target:  Objects  SetTarget

- Select the convex hull for the cluster within the map window

- Cut the cluster road segments:  Objects  Erase Outside, click OK

He then saved all changes to the “WCRds_Cluster table.”  Next, Mr. Dippon computed the total road mileage per cluster.  For this, he performed the following SQL query:

Select Cluster, Sum(ObjectLen(Obj, “mi”))

From WCRds_Cluster

Group By Cluster

Finally, Mr. Dippon compared the road mileage per cluster to HM 5.3’s estimated distribution route distance in a cluster.  He obtained these data from column BO of the “calculations tab” of the distribution module.

Prepared by:	Christian M. Dippon, Senior Consultant, NERA

The values in this workbook are used solely to illustrate the process and are not meant as a proposal of an actual rate. In its May 12, 2004 Recurring Cost Panel Rebuttal testimony, Verizon NW stated that its switching cost studies contained an accidental overstatement of the DS-1 Umbilical Links, which will be corrected in the studies submitted in the compliance phase of this proceeding.