NPA/NXX CODE ACTIVATION Task List

| Assoc. Task # | Process |
| --- | --- |
| 1 | Code Requestor issues Part 1 request to NANPA |
| 2 | Grant requested (NPA-NXX) code or assign new one |
| 3 | Prepare Part 3 and return to Code Requestor |
| 4 | Code Requestor (or contracted AOCN) inputs into the Local Exchange Routing Guide |
| 5 | Code Requestor sends NPA NXX Code Request Supplement Form to the U S WEST Routing Group and a copy to the U S WEST Account Team (email to: routing@sa.uswc.com, or fax to 303-707-9578) |
| 6 | Routing Group pulls Telcordia Data Base RDBS and BRIDS assignment information (LERG/DAP report) |
| 7 | Routing Group runs DRD on all codes |
| 8 | Routing Group verifies information and associates with the NPA/NXX Code Request Supplement Form forwarded by the Code Requestor |
| 9 | Routing Group verifies that CLEC has necessary trunks to route NXX code |
| 10 | If trunking is available, issues routing order to the Load and Resource Manager (LRM) |
| 11 | If no trunking is available, notify the Account Team and issue a preliminary routing order to the Load and Resource Manager |
| 12 | Contact the CLEC to discuss trunking needs, negotiate new code turn-up date |
| 13 | CLEC determines trunking needs, issues orders, negotiates due date |
| 14 | Service Delivery Coordinator receives order for trunks, process in EXACT. Transmit copy of EXACT screen to LRM for translations and routing information |
| 15 | Load and Resource Manager schedules/load to translations group based on Due Date and load requirements |
| 16 | Complex Translations turn up new code on customer requested due date and provides completion notice back to routing group |