Signaling System 7 Provisioning Task List

| Assoc. Task # | Process |
| --- | --- |
| 1 | CLEC issues ASR. Pre-certification for interoperability completed. |
| 2 and 3 | SS7 request is reviewed and determination made about new or addition to existing service. If link request only, Word Doc is issued. |
| 4 | If new SS7 service request, the specific type of service dictates next steps. |
| 5, 6 & 7 | If non-TCAP and ISUP only, translations for service are arranged for ISUP signaling format to existing network trunking. |
| 8 & 9 | If TCAP service, translator builds TCAP assignments in STPs. |
| 10 | Testing with STPs completed; Network Channel (NC) and Network Channel Interface verified. |
| 11 | Switch Operations completes order in system and notifies customer. |
| 12, 13, 14 | If service request is for database access (LIDB, ICNAM or 800 service), translations are built in STPs for global networking. If unable to work provisioning request, forwarded back to Design Services for resolution with CLEC. |
| 15 | Test; refer to translator for corrective action, if necessary. When completed, arrange for coordinated testing with customer for acceptance and final turn-up of the service. |
| 16, 17 18, 19 & 20 | If accepted, make final translations in network and entry of global title information to administrative systems. |
| 21, 22, & 23 | Notify field that translations are ready for input. Perform any physical field work necessary to activate service. |
| 24 | Install translations into STPs. |
| 25 | Build Access 7 translations for recording of message detail. |
| 26 & 27 | Test with customer the access to databases and accuracy of messaging. |
| 28 & 29 | Notify Network Monitoring centers of link activation for surveillance purposes. Complete any additional acceptance testing with customer specific to the application(s) or service request. |
| 30 | Complete service request in provisioning and maintenance systems for administration and billing purposes. |

Signaling System 7 Repair Task List

| Assoc. Task # | Process |
| --- | --- |
| 1 | Customer opens trouble ticket through system interface or contacts Account Management Center to report SS7 trouble. Testing by Customer may or may not have taken place. (And/or) Network Monitoring Alarms (NMA) issues trouble ticket |
| 2 | Center receives the trouble ticket and begins processing ticket. Ticket is analyzed to determine if trouble is related to hardware. |
| 3 | If trouble is detected through network monitoring by U S WEST or other diagnostic tools, customer is notified that U S WEST is working on trouble. |
| 4 | Testing determines trouble location. If trouble exists in U S WEST network, trouble is repaired and the customer is notified. Close out NMA ticket. If trouble is not in U S WEST portion of the circuit. Trouble ticket is closed after customer notification. |
| 5 & 6 | If trouble is determined to be in U S WEST network, trouble analysis continues. |
| 7 | Open ticket in Work Force Administration system. |
| 8 | WFA system assigns trouble case to Design Services Center for further testing. |
| 9 | If link is isolated, check translations and retranslated, if necessary. If link problem is resolved, close trouble ticket. Notify customer. |
| 10, 11, 12, & 13 | If design problem with circuit, issue redesign to meet performance standards. If design problem resolved, close the ticket and notify the customer. If not corrected, update WFA ticket and dispatch to Field. File Abnormal Condition Report on Link Trouble. |
| 14 | Field dispatch to repair physical circuit. Status to customer and estimated time to repair is made. Corrective action taken. |
| 15 | When problem is corrected, customer is notified to coordinate testing and acceptance of circuit operations. Ticket is closed after concurrence by customer. |

AIN Platform (AASCE) Provisioning Task List

| Assoc. Task # | Process |
| --- | --- |
| 1 | Customer issues new service request. |
| 2 | The designated U S WEST Account Manager receives, evaluates and forwards the service request on to Product Management. |
| 3 | Product Management receives the request from the Account Team and processes the order/contract. |
| 4 | Legal Department reviews the request and provides legal/contract support. |
| 5 | Product Management issues request for feasibility pricing. |
| 6 | Advanced Technologies and Capacity Provisioning reviews the request and determines feasibility analysis. |
| 7 | Product Management prepares feasibility response and delivers the response to the Account Manager. |
| 8 | Product Management initiates BART bill for the feasibility preparation work. |
| 9 | Account Manager delivers the feasibility response contract to the customer. |
| 10 | Customer reviews contract and authorizes activities to proceed. |
| 11 | Advanced Technologies develops functional requirements. |
| 12 | Capacity Provisioning develops the Network Planning Documentation and reviews capacity requirements. |
| 13 | Account Team delivers the functional requirements to the customer. The customer verifies the functional requirements. Account Team/Product Management reviews and confirms the customers verified functional requirements. Product Management initiates process, ordering, billing, and estimate development teams. |
| 14 | Advanced Technologies designs, develops, and unit tests code procedures. |
| 15 | Advanced Technologies, Capacity Provisioning, NROC, AIN Lab performs joint effort with product testing and analysis. |
| 16 | NROC/AIN Lab approve for service. |
| 17 | Account Team notifies customer of the completion and provides ordering procedures. |
| 18 | ICSC initiates BART billing. |

AIN Platform (AAOS) Provisioning Task List

| Assoc. Task # | Process |
| --- | --- |
| 1 | Customer issues AIN Service Request (issues AIN Local Service Request to ICSC and E-Mails request to AIN Lab). |
| 2 | ICSC receives Local Service Request from customer. ICSC evaluates the request and resolves any issues with the customer. LSR is logged into the system and forwarded on to the ICSC Screener. |
| 3 | ICSC establishes the Due Date and the Firm Order Confirmation, updates the system, and creates the order. If Due Date is not satisfactory, SDC initiates the Escalation Process. |
| 4 | ICSC issues the request to the AIN Lab. |
| 5 | AIN Lab receives the service request from the ICSC. AIN Lab receives the E-Mail spread sheet of LSR information from the customer and matches the E-Mail request to the request from the ICSC. |
| 6 | AIN Lab begins the provisioning process. AIN Lab notifies NROC to activate SS7 Point Code Translations. AIN Lab translates/mediates the request. |
| 7 | NROC translates SS7 Point Codes. |
| 8 | AIN Lab provisions the AIN Database and April Systems. |
| 9 | NROC/AIN Lab notify ICSC of completion of service activation. |
| 10 | ICSC notifies the customer of the completion. |
| 11 | ICSC initiates BART billing. |

AIN Platform (AAOS/AASCE) REPAIR Task List

| Assoc. Task # | Process |
| --- | --- |
| 1 | Customer reports AIN trouble (service and/or network related). |
| 2 | AMSC receives and evaluates the trouble report. |
| 3 | AMSC creates the trouble ticket. |
| 4 | AMSC hands off trouble ticket to the AIN Lab. |
| 5 | AIN Lab receives and evaluates the trouble ticket. |
| 6 | NROC receives and evaluates the network related trouble. |
| 7 | NROC, AIN Lab resolves the trouble and closes out the trouble ticket. If joint involvement, the lead organization is responsible to close the trouble ticket as required. |
| 8 | NROC and AIN Lab notifies the customer of completion of the trouble ticket. |

LIDB/ICNAM Task List

| Assoc. Task # | Process |
| --- | --- |
| 1 | Customer signs agreement with U S WEST for LIBD and/or ICNAM database services. |
| 2 | U S WEST trains Customer on format to send data and performs test with customer. |
| 3 and 4 | Customer e-mails Data (ASCII) file to System Administrator. |
| 5 and 6 | System Administrator forwards ASCII file to Line Validation Administration System (LVAS). |
| 7 | LVAS loads data into LIBD. |
| 8 or 9 | Data request is closed or customer is notified that data failed to load. |