1

2

3

4

7

6

5

Unbundled Loop

ordered on DLC

systems

Is existing

System UDLC?

Are

existing UDLC or

copper pairs available?

Is IDLC

System INA

capable?

Is this

IDLC system

ISC303?

Utilize appropriate

cards for UBL circuit

or Line & station

transfer (LST) to

copper facilities for

UBL installation

Line & station

transfer (LST) as

required for UBL

installation

Establish, augment

or use existing INA

Digroup to install

UBL circuit

Create/use

Universal

Digroup D?

Is a

1/0 DCS

existing in

C.O.?

Unbundled Loop

using 1/0 DCS

INA capability is only possible in

systems with Time Slot Interchange

(TSI) components.

Yes

Yes

Yes

Yes

Yes

Yes

No

No

No

No

No

No

Note:

As a last resort, Hairpins can be used to

complete the installation of UBLs in very

small quantities (3 loops or less).  This

method has been used by Qwest in the past

and remains a very difficult method, although

not recommended due to several severe

administrative complications.  Use of Hairpins

requires Director level approval to proceed.

Yes

No

Unbundled Loop using Universal Digroup D

Unbundled Loop

using appropriate

COT equipment

(i.e., LS2000, SLC96, SER5

1

2

3

4

5

6

7

Unbundled Loop

using Universal

Digroup D

Unbundled Loop

using Hairpins, not

to exceed 3 at CO.

Is a

total UBL

demand >3

circuits at

CO?

Note:

Manufacture Discontinued

(MD) COT equipment should

be obtained from Reuse stock