Valve Spacing and Rapid Shutdown

Companies must have  the ability to identify  and respond to reportable releases  and  monitor internal pressure and flow rates to minimize spills.  A company must consider terrain, geohazards, drainage and the condition  of the pipe when determining the location and type of valve and their shut-off time.  Whenever a new rapid shutdown valve is to be installed, a surge analysis must be conducted to ensure that the surge pressure in the pipeline will not exceed 110 percent of the MOP as a result of a rapid valve closure.