    - Valve Spacing and Rapid Shutdown  (1) Companies must have procedures to rapidly locate and isolate reportable releases from a pipeline.

(2)  When determining the type of valve to be used, its location, and its shut-off time,  Companies must consider the following:

- terrain,

- geohazards,

- drainage, and

- type and condition of the pipe.

(3)  Whenever a new rapid shutdown valve is to be installed, a surge analysis must be conducted to ensure that the surge pressure in the pipeline will not exceed 110 percent of the Maximum Operating Pressure as a result of a rapid valve closure.