Table 3-5: Point-to-Point Non-Loaded NC and NCI Code Combinations

|  | NCI Code |  |  |
| --- | --- | --- | --- |
| NC Code | U S WEST CO-NI | End-User EU-NI | Description: Application of U S WEST’s Unbundled loop |
|  | Digital Data service transport Interfaces |  |  |
| LX-N | 04QB5.00 | 04DU5.24 | 2.4 kbit/s, not DS0A Level signal |
| LX-N | 04QB5.00 | 04DU5.24S | 2.4 kbit/s, with secondary chan., not DS0A Level signal |
| LX-N | 04QB5.00 | 04DU5.48 | 4.8 kbit/s, not DS0A Level signal |
| LX-N | 04QB5.00 | 04DU5.48S | 4.8 kbit/s, with secondary chan., not DS0A Level signal |
| LX-N | 04QB5.00 | 04DU5.96 | 9.6 kbit/s, not DS0A Level signal |
| LX-N | 04QB5.00 | 04DU5.96S | 9.6 kbit/s, with secondary chan., not DS0A Level signal |
| LX-N | 04QB5.00 | 04DU5.19 | 19.2 kbit/s, not DS0A Level signal |
| LX-N | 04QB5.00 | 04DU5.19S | 19.2 kbit/s, with secondary chan., not DS0A Level signal |
| LX-N | 04QB5.00 | 04DU5.56 | 56.0 kbit/s, not DS0A Level signal |
| LX-N | 04QB5.00 | 04DU5.56S | 56.0 kbit/s, with secondary chan., not DS0A Level signal |
| LX-N | 04QB5.00 | 04DU5.64 | 64.0 kbit/s, not DS0A Level signal |

Table 3-6: Unbundled xDSL Loop NC/NCI Code Combinations

|  | NCI Code |  |  |  |
| --- | --- | --- | --- | --- |
| NC Code | U S WEST CO-NI | End-User EU-NI |  | Description: Application of U S WEST’s Unbundled loop |
| digital SUBSCRIBER line (BASIC Rate ISDN -- DSL) Compatible |  |  |  |  |
| LX-N | 02QC5.OOS | 02IS5.N |  | Digital subscriber Line with 2B1Q Signaling Format Compatible Loop |
| High-bit-rate digital SUBSCRIBER line (HDSL) Compatible |  |  |  |  |
| LX-N | 02QB9.00H | 02DU9.00H |  | HDSL Compatible Loop, Metallic Facility ONLY per ANSI T1E1 Technical Report Number 28 |
| LX-N | 04QB9.00H | 04DU9.00H |  | HDSL Compatible Loop, Metallic Facility ONLY per ANSI T1E1 Technical Report Number 28 |
| ASYMMETRIC digital SUBSCRIBER line (ADSL) Qualified |  |  |  |  |
| LXR- | 02QB5.00A | 02DU5.00A |  | Revised Resistance Design (RRD) Loop that is ADSL qualified with ANSI T1.413 DMT Signaling Format |
| LXR- | 02QB5.01A | 02DU5.01A |  | RRD Loop that is ADSL qualified with ANSI T1.413 DMT Signaling Format and one POTS Channel |
| LXR- | 02QB5.00C | 02DU5.00C |  | RRD Loop that is ADSL qualified with CAP Signaling Format |
| LXR- | 02QB5.01C | 02DU5.01C |  | RRD Loop that is ADSL qualified with CAP Signaling Format and one POTS Channel |
| Note: See Chapter 6 for detailed description of ADSL Loop Qualification. |  |  |  |  |

Table 3-7 lists NC/NCI Code combinations for Digital Unbundled Loops

Table 3-7: Digital Unbundled Loop NC/NCI Code Combinations

|  |  | NCI Code |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NC Code |  | U S WEST CO-NI |  | End-User EU-NI |  | Description |  |
| 160 kbit/s dIGITAL sUBSCRIBER lNE (DSL) |  |  |  |  |  |  |  |
| AD-- | 02QC5.OOS |  | 02IS5.N |  | Digital subscriber Line with 2B1Q Signaling Format, NT function at EU |  |  |
| AD-- | 02QC5.OOV |  | 02IS5.L |  | Digital subscriber Line with 2B1Q Signaling Format, LT function at EU |  |  |
| ADU- | 02QC5.OOS |  | 02IS5.N |  | xDSL-I, 2B1Q Signaling Format, NT function at EU |  |  |
| ADU- | 02QC5.OOV |  | 02IS5.L |  | xDSL-I, 2B1Q Signaling Format, LT function at EU |  |  |
| 1.544 Mbit/s DS1 (also see 77200 & 77375) |  |  |  |  |  |  |  |
| HC-- | 04QB9.11 |  | 04DU9.BN |  | SF Format PER TR-NPL-000342, AMI, without Line Power (w/o LP) |  |  |
| HCD- | 04QB9.11 |  | 04DU9.1KN |  | ANSI ESF, AMI, w/o LP |  |  |
| HCE- | 04QB9.11 |  | 04DU9.1SN |  | ANSI ESF, B8ZS, w/o LP |  |  |
| HCF- | 04QB9.11 |  | 04DU9.CN |  | NON-ANSI ESF, AMI, w/o LP |  |  |
| HCG- | 04QB9.11 |  | 04DU9.SN |  | NON-ANSI ESF, B8ZS, w/o LP |  |  |
| HCJ- | 04QB9.11 |  | 04DU9.AN |  | Free Framing, B8ZS, w/o LP |  |  |
| HCZ- | 04QB9.11 |  | 04DU9.DN |  | SF, B8ZS, w/o LP |  |  |
| 44.736 Mbit/s DS3 (also see 77324) |  |  |  |  |  |  |  |
| HF-- | 04QB6.33 |  | 04DU6.44R |  | DS3 M-frame structured signal. It is an unchannelized signal application, supporting a user payload of 44.210 Mbit/s per ANSI T1.107-1995. |  |  |
| HF-- | 04QB6.33 |  | 04DU6.44 |  | DS3 M-frame structured signal with M23 Multiplex format and 28 DS1 Channels application per ANSI T1.107-1995. |  |  |
| HFC- | 04QB6.33 |  | 04DU6.44A |  | DS3 M-frame structured signal with C-bit Parity application. It is an unchannelized signal application, supporting a user payload of 44.210 Mbit/s per ANSI T1.107-1995. |  |  |
| HFC- | 04QB6.33 |  | 04DU6.44I |  | DS3 M-frame structured signal with C-Bit Parity application and 28 DS1 Channels per ANSI T1.107-1995. |  |  |