| # Line Sharing/Shared Loop Maintenance Task List |  |  |
| --- | --- | --- |
| Assoc. Task # | Process |  |
| 1 | End user customer experiences voice problem with service and calls U S West. |  |
| 2 | End user customer experiences data problem with service and calls CLEC |  |
| 3 | CLEC determines if trouble is voice or facility problem. If voice trouble CLEC refers end user to U S WEST |  |
| 4 | CLEC determines trouble to be related to the data equipment. |  |
| 5 | CLEC resolves own data equipment trouble with the end user and closes their repair ticket with end user. Go to task 21. |  |
| 6 | The Repair Call Handling Center (RCHC) determines line sharing customer from records and refers either the end user or the CLEC to a special number in the AMSC. |  |
| 7 | The Repair Call Expert (RCE) determines if the trouble is data, voice, or both data and voice. |  |
| 8 | RCE finds trouble is not associated with U S WEST. |  |
| 9 | RCE refers end user customer to their CLEC and closes trouble ticket. |  |
| 10 | RCE determines a line translation trouble related to U S WEST. |  |
| 11 | RCMAC clears line translation trouble, contacts customer and closes trouble ticket. |  |
| 12 | RCE determines trouble is software related. |  |
| 13 | Complex Translations repairs software trouble, contacts customer and closes trouble ticket. |  |
| 14 | RCE determines problem is physical trouble. |  |
| 15 | LRAC schedules and loads work to technician(s). |  |
| 16 | Central office trouble loaded to Central Office Technician (COT) via WFA/DI |  |
| 17 | COT determines if "wire around" required |  |
| 18 | If "wire around" not required COT repairs central office trouble, contacts customer and closes trouble ticket. |  |
| 19 | Outside trouble is loaded to LNO I&M Technician |  |

| # Line Sharing/Shared Loop Maintenance Task List |  |  |
| --- | --- | --- |
| Assoc. Task # | Process |  |
| 20 | Outside technician clears trouble, contacts customer and closes ticket |  |
| 21 | CLEC reports data related trouble to U S WEST or requests MLT test be performed to isolate data/facility. |  |
| 22 | RCE takes report |  |
| 23 | RCE screens trouble report and determines need to check physical facilities. |  |
| 23.1 | Determine results of MLT test. If Central Office trouble go to task 15. If not Central Office trouble go to task 24. |  |
| 24 | LRAC loads trouble ticket via WFA/DO to outside technician |  |
| 25 | LRAC dispatches able technician with test set. |  |
| 26 | Outside technician performs cooperative testing to isolate trouble |  |
| 27 | Outside tech verifies facilities are good and closes ticket |  |
| 28 | CLEC performs cooperative testing the U S WEST field technician. |  |
| 29 | CLEC determines whether facilities can continue to support line sharing |  |
| 30 | Facilities can support line sharing. Resolve facility issues with U S WEST. |  |
| 31 | Facilities can no longer support line sharing. CLEC to disconnect line sharing. |  |
| 32 | End user receives notice form CLEC that Line Sharing is unavailable. |  |
| 33 | "Wire Around" ¹ CLEC equipment to restore voice service to end user |  |
| 34 | Call end user and code ticket as "wire around" |  |
| 35 | End user's voice service is restored and data is disconnected. |  |
| 36 | RCHC/AMSC receive trouble tickets coded as "wire around" |  |
| 37 | Contact CLEC as to situation i.e.,"wire around" and close trouble report. |  |

| # Line Sharing/Shared Loop Maintenance Task List |  |  |
| --- | --- | --- |
| Assoc. Task # | Process |  |
| 38 | CLEC receives notification of data equipment problem. |  |
| 39 | CLEC investigates and isolates data trouble. |  |
| 40 | CLEC contact ISC to process a service order to re-establish line sharing. |  |

Note 1: If an end-user complains of a voice problem that may be related to the use of the shared line for data services, CLEC and ILEC will work together and with the end-user to solve the problem to the satisfaction of the end-user.  ILEC will not disconnect the data service without the written permission of the CLEC unless the end-user’s voice service is so degraded that the end-user cannot originate or receive voice grade calls. (per 14 State Line Sharing Agreement 4/24/00, Arizona, Colorado, Idaho, Iowa, Montana, Nebraska, New Mexico, North Dakota, Oregon, South Dakota, Utah, Washington, and Wyoming.)