### OPERATOR INSPECTION-SPECIFIC INFORMATION

| ##### Inspection Date(s): |  | ##### 11-16-2004 through 11-18-2004 |  |  |
| --- | --- | --- | --- | --- |
| Name of Operator: |  | Weyerhaeuser Longview Mill |  |  |
| OPS Operator ID: |  |  |  |  |
| State/Other ID: |  | WA |  |  |
| H.Q. Address: 3401 Industrial Way Longview, WA 98632-7117 |  |  | Company Official: | Ron Kosloski |
|  |  |  | Title: | Facilities Engineer |
|  |  |  | ##### Phone Number: | (360) 425-2150 |
|  |  |  | Fax Number: |  |
| Web Site: |  |  | Email Address: | ronkosloski@weyerhaeuser |
| Employees Covered by OQ Plan: |  |  | 0 |  |
| Contractors Covered by OQ Plan: |  |  | 5 |  |
| Total Mileage Represented: |  |  | 10. |  |

| Persons Interviewed | Title | Phone Number | Email Address |
| --- | --- | --- | --- |
| Steve Oxford | Engineer with Industrial Gas Service | (303) 422-3400 | igsinc@att.net |
| Ron Kosloski | Facilities Engineer | (360) 425-2150 | ronkosloski@weyerhaeuser.com |

To add rows, press TAB with cursor in last cell.

| OPS/State Representatives | Region/State |
| --- | --- |
| Patti Johnson | WA State |

To add rows, press TAB with cursor in last cell.

Remarks:

Mileage Covered by OQ Plan (by Company and State)

List each company and subsidiary separately, broken down by state (using 2-letter designation).  If  a company has intrastate and/or interstate mileage in several states, use one row per state.  If there are both gas and liquid lines, use both the first and second table. For small gas operators (e.g. master meter, LP), use the third table.

Jurisdictional to Part 192 (Gas) Mileage

| Company (Gas Operator) | Operator ID | State | Interstate Gathering | Intrastate Gathering | Interstate Transmission | Intrastate Transmission | Interstate Distribution | Intrastate Distribution | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 10. |  |  |  |

(To add rows, press TAB with cursor in last cell.)

Jurisdictional to Part 195 (Hazardous Liquid) Mileage

| Company (Liquid Operator) | Operator ID | State | Interstate Transmission | Intrastate Transmission | Remarks |
| --- | --- | --- | --- | --- | --- |

(To add rows, press TAB with cursor in last cell.)

Jurisdictional to Part 192 (Gas) Mileage – Small Operators

| Company (Small Gas Operator) | Operator ID | State | Small Gas (e.g., master meter) | LP | Remarks |
| --- | --- | --- | --- | --- | --- |

(To add rows, press TAB with cursor in last cell.)

- Supply company name and Operator ID, if not the master operator from the first page (i.e., for subsidiary companies).

- Use OPS-assigned Operator ID.  Where not applicable, leave blank or enter n/a.

- Use only 2-letter state codes in column #3, e.g., TX for Texas.

- Enter number of applicable miles in all other columns.  (Only positive values.  No need to enter 0 or n/a.)

1 - Document Program Plan, Implementing Procedures and Qualification Criteria

1.01 Does the operator’s plan identify covered tasks and does it specify task-specific reevaluation intervals for individuals performing covered tasks? [Enforceable]

| No Issues Identified | Inspection Notes: Weyerhausuers plan lists task for individual employees. There are no task procedures included in the plan, some procedures are in the O&M manual. Weyerhaeuser's OQ plan refers to Section 2 of O&M manual for Abnormal operating conditions (AOC) but Section 2 does not address AOCs. Abnormal Operating Conditions are treated as a pipeline incident. Weyerhaeuser OQ plan does not include reevaluation intervals. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

1.02 Does the operator employ contractor organizations to provide individuals to perform covered tasks?  If so, what are the methods used to qualify these individuals and how does the operator ensure that contractor individuals are qualified in accordance with the operator’s OQ program plan?

* Verify that the operator’s written program includes provisions that require all contractor and subcontractor individuals be evaluated and qualified prior to performing covered tasks, unless the covered task is performed by a non-qualified individual under the direction and observation of a qualified individual. [Enforceable]

| No Issues Identified | Inspection Notes: Weyerhaeuser is the operator of the pipeline, however Weyerhaeuser employs Industrial Gas Service to operate the pipeline. Initial qualification were based on Work Performance History review but not documented. Weyerhaeuser's plan does not include evaluation methods for covered tasks or AOC's or reevaluation intervals Weyerhaeuser plan does not address contractor employees evaluations and qualifications. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

1.03 Has the operator’s OQ program included provisions that require individuals from any other entity performing covered task(s) on behalf of the operator (e.g., through mutual assistance agreements) be evaluated and qualified prior to task performance?

* Verify that other entities that perform covered task(s) on behalf of the operator are addressed under the operator’s OQ program and that individuals from such other entities performing covered tasks on behalf of the operator are evaluated and qualified consistent with the operator’s program requirements. [Enforceable]

| No Issues Identified | Inspection Notes: No mutual assistance agreements. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

1.04 Does the operator’s OQ program plan contain policy and criteria for the use of training in initial qualification of individuals performing covered tasks, and are criteria in existence for re-training and re-evaluation of individuals if qualifications are questioned? [Non-Enforceable]

| No Issues Identified | Inspection Notes: No training or retraining addressed in OQ plan other than emergency drills. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

1.05 Did the operator meet the OQ Rule requirements for establishing a written operator qualification program and completing qualification of individuals performing covered tasks?

* Verify that the operator’s written qualification program was established by April 27, 2001. [Enforceable]

* Verify that the written qualification program identified all covered tasks for the operator’s operations and maintenance functions being conducted as of October 28, 2002. [Enforceable]

* Verify that the written qualification program established an evaluation method(s) to be used in the initial qualification of individuals performing covered tasks as of October 28, 2002. [Enforceable]

* Verify that all individuals performing covered tasks as of October 28, 2002, and not otherwise directed or observed by a qualified individual were qualified in accordance with the operator’s written qualification program. [Enforceable]

| No Issues Identified | Inspection Notes: October 28,2002 a list of employee tasks was developed. The title of each covered task in the OQ plan was the CFR 192 code # and associated title. A list of duties and some tasks required to to preform 192 requirements was attached. The list did not include all covered tasks i.e. excavtion, calibration and operation of leak detection and locating equipment & inspection of exposed pipe. Covered task list not complete and no documentaiton of four part test being applied. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

2 - Identify Covered Tasks and Related Evaluation Methods

2.01 How did the operator develop its covered task list?

* Verify that the operator applied the four-part test to determine whether 49 CFR Part 192 or 49 CFR Part 195 O&M activities applicable to the operator are covered tasks.[Enforceable]

* Verify that the operator has identified and documented all applicable covered tasks.[Enforceable]

| No Issues Identified | Inspection Notes: No documentation that the four part test was applied. All covered task no identified. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

2.02 Has the operator established and documented the evaluation method(s) appropriate to each covered task?

* Verify what evaluation method(s) has been established and documented for each covered task.[Enforceable]

* Verify that the operator’s evaluation program ensures that individuals can perform assigned covered tasks. [Enforceable]

| No Issues Identified | Inspection Notes: No evaluation methods establsihed, no documention of evaluations. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

3 - Identify Individuals Performing Covered Tasks

3.01 Does the operator’s program document the evaluation and qualifications of individuals performing covered tasks, and can the qualification of individuals performing covered tasks be verified at the job site?

* Verify that the operator’s qualification program has documented the evaluation of individuals performing covered tasks. [Enforceable]

* Verify that the operator’s qualification program has documented the qualifications of individuals performing covered tasks. [Enforceable]

| No Issues Identified | Inspection Notes: No evaluation methods established, no evaluations and qualification documentation. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

3.02 Has the operator established provisions to allow non-qualified individuals to perform covered tasks while being directed and observed by a qualified individual, and are there restrictions and limitations placed on such activities?

* Verify that the operator's program includes provisions for the performance of a covered task by a non-qualified individual under the direction and observation by a qualified individual. [Enforceable]

| No Issues Identified | Inspection Notes: No provision in written plan for non qualified individuals to perform covered tasks while being directed and observced by a qualified individual. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

4 - Evaluate and Qualify Individuals Performing Covered Tasks

4.01 Does the operator use work performance history review as the sole method of qualification for individuals performing covered tasks prior to October 26, 1999, and does the operator's program specify that work performance history review will not be used as the sole method of evaluation for qualification after October 28, 2002?

* Verify that after October 28, 2002, work performance history is not used as a sole evaluation method. [Enforceable]

* Verify that individuals beginning work on covered tasks after October 26, 1999 have not been qualified using work performance history review as the sole method of evaluation. [Enforceable]

| No Issues Identified | Inspection Notes: No documentation. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

4.02 Are all qualified individuals able to recognize and react to AOCs? Has the operator evaluated and qualified individuals for their capability to recognize and react to AOCs? Are the AOCs identified those that the individual may reasonably anticipate and appropriately react to during the performance of the covered task? Has the operator established provisions for communicating AOCs for the purpose of qualifying individuals?

* Verify that individuals performing covered tasks have been qualified in recognizing and reacting to AOCs they may encounter in performing such tasks. [Enforceable]

| No Issues Identified | Inspection Notes: AOC not identified and no evaluation method for AOCs, no evaulation for AOCs and no documention for AOCs. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

5 - Continued/Periodic Evaluation of Individuals Performing Covered Tasks

5.01 Does the operator's program include provisions to evaluate an individual if the operator has reason to believe the individual is no longer qualified to perform a covered task based on: Covered task performance by an individual contributed to an incident or accident; Other factors affecting the performance of covered tasks?

* Verify that the operator's program ensures evaluation of individuals whose performance of a covered task may have contributed to an incident or accident. [Enforceable]

* Verify that the operator has established provisions for determining whether an individual is no longer qualified to perform a covered task, and requires reevaluation. [Enforceable]

| No Issues Identified | Inspection Notes: Written OQ plan does not have provision for determining whether an individual is no longer qualifed to perform a covered task. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

5.02 Has the operator established and justified requirements for reevaluation of individuals performing covered tasks?

* Verify that the operator has established intervals for reevaluating individuals performing covered tasks. [Enforceable]

| No Issues Identified | Inspection Notes: No reevaluation intervals established. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

6 - Monitor Program Performance; Seek Improvement Opportunities

6.01 Does the operator have provisions to evaluate performance of its OQ program and implement improvements to enhance the effectiveness of its program? [Non-Enforceable]

| No Issues Identified | Inspection Notes: No self audit procedures. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

7 - Maintain Program Records

7.01 Does the operator maintain records in accordance with the requirements of 49 CFR 192, subpart N, and 49 CFR 195, subpart G, for all individuals performing covered tasks, including contractor individuals?

* Verify that qualification records for all individuals performing covered tasks include the information identified in the regulations. [Enforceable]

* Verify that the operator's program ensures the retention of records of prior qualification and records of individuals no longer performing covered tasks for at least five years. [Enforceable]

* Verify that the operator's program ensures the availability of qualification records of individuals (employees and contractors) currently performing covered tasks, or who have previously performed covered tasks. [Enforceable]

| No Issues Identified | Inspection Notes: No documentation records. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

8 - Manage Change

8.01 Does the operator's OQ program identify how changes to procedures, tools standards and other elements used by individuals in performing covered tasks are communicated to the individuals, including contractor individuals, and how these changes are implemented in the evaluation method(s)?

* Verify that the operator's program identifies changes that affect covered tasks and how those changes are communicated, when appropriate, to affected individuals. [Enforceable]

* Verify that the operator's program identifies and incorporates changes that affect covered tasks. [Enforceable]

* Verify that the operator's program includes provisions for the communication of changes (e.g., who, what, when, where, why) in the qualification program to the affected individuals. [Enforceable]

* Verify that the operator incorporates changes into initial and subsequent evaluations. [Enforceable]

* Verify that contractors supplying individuals to perform covered tasks for the operator are notified of changes that affect task performance and thereby the qualification of these individuals. [Enforceable]

| No Issues Identified | Inspection Notes: Communication of change not addressed in plan. |
| --- | --- |
| Potential Issues Identified (explain) |  |
| N/A (explain) |  |
| Not Inspected |  |
| Check exactly one box above. |  |

9 – Field Inspection Findings

See the field inspection report.  The person interviewed for the field review was the person who wrote the OQ plan. The individual had explanations of the plan.  However, his explanations were not in the OQ plan

Additional Inspection Notes