| WN U-74 |  |  | First Revision of Sheet No. J.3 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Canceling Original Sheet No. J.3 |  |  |  |  |  |  |  |  |  |  |
| PACIFIC POWER & LIGHT COMPANY |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | FOR COMMISSION’S RECEIPT STAMP |  |  |
|  | GENERAL RULES AND REGULATIONS 8. METERING: (continued) (M) pg j2 (M) pg j2 (K) pg j4 B. METER TEST PROCEDURES: (As of the effective date of this tariff.) (continued) 2. In-Service Meter Performance Testing (continued). The Company's random testing is based on ANSI/ASQC Z1.4 Sampling Procedures and Tables for Inspection by Attributes. Meters that, on test, fail to meet accuracy specifications and that cannot be recalibrated will be replaced, retired, or salvaged within 10 days. The Company's meter replacement and/or retirement programs meet the requirements set forth in ANSI 12.1 Section 5. 3. Meter Tests at Customer Request. Customers may request the Company to test the accuracy of the meter at the Customer's premises. The Company will perform such a test once in twelve months without charge. If the Customer requests more than one test in twelve months, the Company may request the amount specified in Schedule 300. If the meter errs more than 2%, the Company will refund the advance. 4. Billing Adjustments for Meter Error. If a meter, tested at a Customer's request, errs more than +/- 2%, slow or fast, the Company will adjust its billings. The Company will correct billings for the six months prior to the test, or to the date of the last test, whichever is the shortest time. The Company may adjust from the date the error occurred, if it can determine that date. The Company will refund only to the customer last served by the meter prior to the test. If a meter fails to correctly register the amount of electric power or energy used by the customer, the Company will estimate the amount of such use from the best available information. 5. Watthour Reference Standards. The Company uses a precision reference standard as primary calibration for meter testing equipment. Meter test equipment is examined annually to assure compliance with the National Institute of Standard and Technology (NIST) standards. The certification interval for PacifiCorp's other standards, are as follows: |  |  |  |  |  |  |  |  |  |  |  |  |
| (Continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Issued |  |  |  |  | February 6, 2008 |  | Effective |  |  | March 7, 2008 |  |  |  |
| Issued by |  |  |  | PACIFIC POWER & LIGHT COMPANY |  |  |  |  |  |  |  |  |  |
| By |  | Andrea L. Kelly |  |  |  |  |  | Title | Vice President, Regulation |  |  |  |  |
|  |  | TF2 J.3REV |  |  |  | Case No. |  |  |  |  |  | UE- |  |
| Form F |  |  |  |  |  |  |  |  |  |  |  |  |  |