Exhibit ___ (RW-1T)

Docket No.  UT-031472

Witness: Robert Williamson

BEFORE THE WASHINGTON UTILITIES AND TRANSPORTATION COMMISSION

| WASHINGTON EXCHANGE CARRIER ASSOCIATION, et. al., Petitioners, v. LOCALDIAL CORPORATION, an Oregon Corporation, Respondents. | DOCKET NO. UT-031472 |
| --- | --- |

## DIRECT TESTIMONY OF

## Robert Williamson

## STAFF OF

## WASHINGTON UTILITIES AND

## TRANSPORTATION COMMISSION

# February 27, 2004

Q.	Please state your name and business address.

A.	My name is Robert Williamson, and my business address is 1300 South Evergreen Park Drive Southwest, P.O. Box 47250, Olympia, Washington, 98504.  My business e-mail address is bwilliam@wutc.wa.gov.

### Q.	By whom are you employed and in what capacity?

A.	I am employed by the Washington Utilities and Transportation Commission as a Utility Engineer in the Telecommunications Section.

Q.	Please state your qualifications to provide testimony in this proceeding.

A.	I have provided a description of my qualifications as Exhibit ____ (RW-2).

Q.	Have you presented testimony before this Commission in other cases?

A.	Yes I have.  I testified in Docket UT-011439 concerning a petition for a line extension waiver in Eastern Washington and in Docket UT-030614 concerning a petition by Qwest for competitive classification.

Q.	What is the purpose of your testimony?

A.	This case concerns the regulatory classification of a service that the Respondent,

LocalDial, characterizes as “a supplemental phone service for domestic long distance calling.”  LocalDial has also characterized its service as a Voice over Internet Protocol (VoIP) service.  Therefore, in the first part of my testimony, I provide some background information to help the Commission understand what VoIP means and how it compares with telephony that utilizes the Public

Switched Telephone Network (PSTN).  There are a number of different services that may be referred to as VoIP.  Because the regulatory classification of those services by the Federal Communications Commission (FCC) is likely to differ, I will also explain the categories of VoIP service that the FCC set out in a 1998 Report to Congress.  In the second section, I describe how LocalDial’s service works, both from the users’ standpoint, and from a technical standpoint.  In the third section, I present my conclusion that the service provided by LocalDial is a telecommunications service within the meaning of Title 80 RCW, and that it may be classified by the Commission as a provider of intrastate interexchange (long distance) services.

I.  Background:

Internet Telephony and the Public Switched Telephone Network

Q.	Please explain briefly what is generally meant by “the Internet.”

A.	The Internet is a grouping of many large, worldwide computer networks joined over high-speed backbone data links.  It began in the late 1960s when the U.S. Department of Defense Advanced Research Projects Agency (ARPA) connected many research organizations in the U.S. over an experimental wide-area

computer network.  The goal was to share scarce computing resources, and many of the government contractors used the network for collaboration by sharing

files, software, and electronic mail (later to become email).  As the network grew from a handful to a network of tens of thousands of hosts, the original ARPAnet became the backbone of regional and local networks that we now know as the Internet.

In 1988 the publicly funded National Science Foundation (NSF) transitioned the Internet to a commercial network (NSFnet) of multiple

backbones run by long distance carriers like MCI, Sprint, and AT&T.  The Internet backbone continues to be provided by multiple carriers, the largest of which are AT&T, MCI, Sprint, Qwest, and Level3.  Today the Internet connects millions of hosts around the world.

It is also helpful to understand the difference between the “Internet” as opposed to “internets.”  The capitalized version (the Internet) is the network that began as the ARPAnet and has grown into a network made up of directly or indirectly connected commercial backbones, and is universally available to the general public.  The lower case “internet” is any network of multiple smaller networks that does not necessarily connect to the Internet, that may or may not use IP as the standard protocol, and that is not generally available to the public.  An “intranet” is a private “internet” used to connect corporate and other institutional users.

Q.	What is a protocol?

A.	A protocol is a rule that guides how an activity should be performed.  When

used in computing, a protocol is a convention or standard that controls or

enables the connection, communication, and data transfer between two

computing devices.  Protocols may be implemented by hardware, software, or a combination of the two.  A number of protocols are used in both the PSTN and the Internet.

Q. 	What is Internet Protocol?

A.	Internet Protocol (IP) is the software protocol on which the Internet is based.  It was developed in the early 1980s and soon became the standard protocol for ARPAnet.  IP keeps track of the Internet’s addresses for different nodes, routes outgoing messages, and recognizes incoming messages.  It allows a "packet” to traverse multiple networks on the way to its final destination.  A packet is a small amount of data sent over a network together with the address of its origin and destination, and information that connects it to the related packets being sent.

Q.	What is Voice over Internet Protocol (VoIP)?

A.	VoIP is a technology that allows voice traffic to be packetized and transported

and routed as data, without the need for the traditional circuit switching of the PSTN.  The data packet traffic (voice) is directed (i.e. routed) to the desired location using IP addressing.  The data packet traffic may be routed over the Internet or a corporate intranet or a combination of both.  The term “Internet Telephony” is sometimes used interchangeably with VoIP even though the voice transmission may never touch the Public Internet.  In all cases Internet Protocol is used.  At the present time, the focus of development for VoIP is to replicate the

basic functionality and quality of the PSTN, but by utilizing IP networks (either the Internet or internets).

Q.	Briefly compare the Internet and the PSTN as media for transmitting voice conversations.

A.	All voice communication starts as natural acoustic compression waves that transmit voice through the air.  When you speak into a telephone handset, a microphone converts these compression waves to an analog electrical signal that represents the acoustical waves with an analog (similar to or analogous) electrical signal.  At some point in the PSTN (such as at the local exchange company’s end office), the analog signal is sampled, most commonly 8000 times a second, then encoded into a binary (digital) number consisting of zeros and ones.  In

computer-to-computer and computer-to-phone VoIP services (discussed in detail below), the same kind of analog sampling occurs on equipment (such as a personal computer or a “SIP” phone) at the user’s location.  In other words, both the PSTN and IP networks transmit voice digitally.

An IP network uses IP addresses (unique, 32 bit numbers for each host on the Internet) to route packets, while the PSTN uses telephone numbers to route calls through circuits.  Unlike the protocols utilized on the PSTN (such as time

division multiplexing or “TDM”), IP was designed to facilitate the transportation of packet data rather than real time voice communications.  With IP there is no practical distinction between voice, video, or data since they are all transmitted within similar packets.

Using the TDM protocol, the PSTN creates dedicated circuits between users through a maze of switches and connecting trunks ensuring sufficient bandwidth between the two users for the length of a call.  Once that call is terminated, the switches release all of the interconnected circuits so that the bandwidth can be used for other calls.  The PSTN is a “connection” oriented network and as such, circuit bandwidth is allocated for the length of the call even if no conversation is taking place.  When all the circuits are in use, subsequent call attempts are blocked and given a busy indication.

In contrast to the PSTN, packet-switching (such as those using IP) networks divide voice (or any other information) into individual packets of

digital bits which are individually transmitted.  The packets may take different routes but ultimately arrive at the destination where they are reassembled.  IP networks are “connectionless”, meaning there is no end-to-end PSTN-like circuit created for each call.  Because IP networks are “connectionless” no calls are blocked as they can be in the “connection” oriented PSTN.  However, as the

traffic load builds, the quality of a voice transmission over an IP network will degrade to the point that conversation is impossible or the call is dropped.  Thus, as mediums for transmitting voice, both the PSTN and IP networks have capacity limitations, though an end-user experiences the consequences of exceeding that limitation in different ways.  Under heavy traffic conditions the PSTN blocks voice traffic by denying access to the network whereas the IP network may degrade voice traffic by causing long delays between transmission and receipt of data packets.

The PSTN uses North American Numbering Plan (NANP) telephone numbers to locate customers, complete and bill calls.  Geographic local dialing areas determine which calls are “free” (i.e., covered by flat rated local service) and which will be billed.  Computer-to-computer and computer-to-phone VoIP rely on IP addresses—which are not based on geography—to locate and

complete a call.  Thus, where telephone numbers are geographic in nature, IP addresses are not.

Q.	Is there more than one type of VoIP?

A.	Yes.  The FCC, in a 1998 Report to Congress on Universal Service (frequently

called the “Stevens Report”) posited three distinct types of VoIP services:

1.)	“Computer-to-computer”, where users on separate computers hold a voice communications session over a packet network using IP.  The

PSTN is never used, nor are telephone numbers from the North American Numbering Plan.  The voice connection may be over the public Internet or

a private IP intranet.  Pulver.com’s Free World Dialup is an example of this type of VoIP.  On February 12, 2004, the FCC declared that Free World Dialup is an unregulated information service.

2.)	“Computer-to-phone” or “phone-to-computer”, which bridges the different worlds of IP and the PSTN.  The ability to connect a voice call between a computer user and a customer on the PSTN became possible

with the advent of IP gateways—computer hardware that IP telephony providers use to convert digital packets originated on an IP network into TDM (the most common protocol used by the PSTN) so that the call can

be completed via the existing PSTN.  The reverse is also possible using an

IP gateway:  a call may be originated on the PSTN and terminated on the public Internet or a private IP intranet.  NANP telephone numbers are

used at both ends of the call.  The service provided by Vonage is an

example of this type of VoIP.

3.)	”Phone-to-phone”, in which the user simply dials a local traditional NANP telephone number using the user’s existing telephone and is then prompted to dial the desired NANP long distance telephone number that the user wishes to be connected to.  The call is transported to an IP gateway, which converts the digital TDM voice representation to digital IP packets and routes the IP packets to another IP gateway where the process is reversed.  The call is completed via the PSTN to the called NANP telephone number.  The call may be routed over the public Internet or a private IP intranet but all calls are originated and terminated on the PSTN.  Because calls using this type of service originate and terminate on the PSTN with some of the transport occurring via IP, this service may be referred to as “IP-in-the-middle.”

Unlike both computer-to-computer and computer-to-phone VOIP, with phone-to-phone VOIP the conversion from TDM to IP and back to TDM is transparent to the originating and terminating customers.  The call is originated using the ordinary touch-tone phone that virtually every residential customer already owns, over the analog loop that is already

connected to most residences, and is terminated to a NANP telephone number just like any long distance call to another analog touch-tone phone.  AT&T’s phone-to-phone VoIP offering is an example of this type of VoIP service.

Q.	Which type of VoIP is employed by the LocaDial?

A.	As I will explain below, LocalDial’s service is phone-to-phone, “IP-in-the-middle” VoIP service.

II.  How LocalDial’s Service Works

Q.	From a customer’s perspective, how does LocalDial’s service work?

A.	LocalDial advertises its service as providing “[u]nlimited long distance calling

for a flat rate.”  “[L]ocalDial is … a supplemental phone service for domestic long distance calling.”

To access LocalDial’s long distance service, the customer dials a local access number issued by LocalDial.  The customer receives a prompt, and then dials the long distance destination telephone number.  The destination telephone number can be any telephone number in the 48 continental United States.  Users

of LocalDial’s phone-to-phone VoIP service find that the service is indistinguishable from traditional telephone long distance service.

Q.	From a technical standpoint, how does LocalDial provide its service?

A.	On its website LocalDial states: “LocalDial uses a Voice-over-IP system that ends your call by means of digitally compressed and encrypted packets over our own private network to bypass long distance tolls” (emphasis added).  In truth, the service originates and terminates switched inter-exchange calls between traditional touch-tone telephones on the PSTN with no net change to form or content.  The only difference between this LocalDial service and traditional long distance service is that LocalDial inserts IP in a small portion of the transmission path and converts it back to TDM before handing the call to the LEC for final delivery as a local call.

LocalDial leases T1 PRI (Primary Rate ISDN) facilities from a competitive local exchange company (CLEC).  The CLEC has a switch in Seattle from which it provides local exchange access services in a number of Qwest wire centers throughout the state.  Through this arrangement, LocalDial is able to provide its customers in many parts of Washington with a telephone number in their local calling area that connects them to LocalDial’s leased T1 trunk facility via CLEC’s

switch.  The T1 trunks connect to LocalDial’s Integrated Access Devices (IAD), Gateways, and routers, which are located at the Westin Building in Seattle.

Calls to and from Spokane and Western Washington as far south as Centralia/Chehalis use the LocalDial Seattle facility at the Westin Building, and calls to and from Western Washington south of Centralia/Chehalis to the Oregon border use a LocalDial facility in Portland.  The leased T1 PRI’s are bundled in DS3’s (28 DS1s or T1s).  The IADs de-multiplex each DS3 into the separate T1 PRIs, which then connect to the Gateways.  The gateway verifies the caller’s number against a database of known subscribers to LocalDial’s service and then prompts the customer to dial the long distance telephone number that they want

to reach.  If the called number is in an area served by the LocalDial network in Washington, the gateway converts the call to IP format and routes the call

packets to the router and a particular IP address.  The call packets are then

routed to the IP address dedicated to the appropriate port associated with the terminating trunk via an internal LAN.  The gateway converts the IP packet format back to TDM and sends the call to the T1 PRI associated with the correct terminating area.  For a call from Seattle to Spokane or from Olympia to Bellingham, this whole process of converting the call from TDM to IP and back to TDM again occurs in the room at the Westin Building.  The T1 PRI terminates in

the CLEC office which, after receipt of the called telephone number, routes the call over local interconnection trunks to the terminating ILEC central office, or intermediate local tandem, as a local call.

Calls that terminate at the Portland facility are sent from the router in Seattle over the Internet to an IP address in Portland.  Some interstate calls and calls that cannot be terminated on the intrastate LocalDial/Competitive Local Exchange Carrier (CLEC) network are sent to Long Distance resellers (approximately four) for termination (access charges are paid on these calls).  Interstate calls that can be completed over the extended LocalDial network are routed over the Internet to the appropriate LocalDial gateways.

Q.	Please explain why the LocalDial network is classified as phone-to-phone VoIP.

A.	The FCC defined phone-to-phone IP Telephony as a service that:  1) holds itself out as providing voice telephony service;  2) does not require the customer to use CPE different from that necessary to place an ordinary touch-tone call over the public switched telephone network;  3) allows the costumer to call telephone

numbers assigned in accordance with the North American Numbering Plan;  4) transmits customer information without net change in form or content.

LocalDial’s “supplemental Long Distance” service passes all of the above tests set forth by the FCC and is a phone-to-phone service.

III.  LocalDial’s Service Is ”Telecommunications”

Under Title 80 RCW And Is The Same For All Practical Purposes,

As Any Long Distance Service That Is Subject To Access Charges

- Does the service offered by LocalDial meet the definition of

“telecommunications” that is set out in Title 80 RCW?

A.	Yes it does.  RCW 80.04.010 defines “telecommunications” as “[t]he transmission of information by wire, radio, optical cable, electromagnetic, or other similar means.  As used in this definition, ‘information’ means knowledge or intelligence represented by any form of writing, signs, signals, pictures, sounds, or any other symbols.”  Under the same statute, “telecommunications company” is defined to include  “[e]very corporation, company … owning, operating or managing any facilities used to provide telecommunications for hire, sale, or resale to the general public within this state.”

The terms of the statute are broad and does not distinguish between technologies.  It seems clear that if the modern PSTN fits this definition of telecommunications (as the “transmission of information by wire, radio, or

optical cable”) then LocalDial’s service should, too.

It is my understanding that LocalDial owns and operates the gateways, servers, and other equipment I mention above and, therefore, Staff concludes

that the company is a “telecommunications company” under the statute.

Q.	What has the FCC said about phone-to-phone IP telephony?

A.	In the 1998 Stevens Report, the FCC stated:  “[W]e note that, to the extent we conclude that certain forms of phone-to-phone IP telephony service are ‘telecommunications services’, and to the extent the providers of these services obtain the same circuit-switched access as obtained by other inter-exchange carriers, and therefore impose the same burdens on the local exchange carriers,

we may find it reasonable that they pay similar access charges.”

The FCC stated that a carrier offering a simple, transparent transmission path, without the capability of providing enhanced functionality, offers telecommunications.   The FCC also noted that certain protocol processing

services that result in no net protocol conversion to the end user are deemed telecommunications services.  “[T]he protocol processing that takes place incident to phone-to-phone IP Telephony does not affect the service’s classification, under the Commission’s current approach because it results in no protocol conversion to the end user.”

The FCC concluded that the nature of the service being offered to customers determined its classification.  If a user can receive nothing more than pure transmission, the service is a telecommunications service;  if the user can receive enhanced functionality, such as manipulation of information and interaction with stored data, the service is an information service.

From a functional standpoint, the FCC found that users of phone-to-

phone IP telephony services (such as those provided by LocalDial) obtain only voice transmission services, rather than information services such as access to stored files.

Q.	Please summarize the reasons why you conclude the phone-to-phone service provided by LocalDial is a telecommunication service and that it may be classified as a provider of intrastate long distance (interexchange) service.

A.	All of the reasons identified by the New York Public Service Commission in a proceeding to determine the regulatory classification of a company providing a service similar to that of LocalDial apply here as well:

1)	LocalDial holds itself out as providing voice telephony service.

2)	LocalDial does not provide enhanced functionality to its customers, such as storing, processing or retrieving information.

3)	LocalDial’s customers are not required to use Customer Provided Equipment (CPE) different from the CPE used to place ordinary calls over the PSTN.

4)	LocalDial’s customers place calls to telephone numbers assigned in accordance with the NANP.

5)	The use of IP is incidental to LocalDial’s own private network and does not result in net protocol conversion to the end user.

6)	LocalDial uses the same circuit switched access as obtained by IXCs and imposes the same burdens on the local exchange as do IXCs.

The phone-to-phone service as provided by LocalDial is a simple, transparent, long distance telephone service, virtually identical to traditional circuit-switched carriers.  LocalDial’s service fits the definition of “telecommunications” contained in RCW 80.04 and the 1998 FCC Stevens Report To Congress.  It is not an “information service” or an “enhanced service”.  Thus, LocalDial’s traffic is access traffic identical to other IXC traffic.

###### Q	Does this complete your testimony?

A.	Yes it does.

www.888localdial.com

In the Matter of Federal-State Joint Board on Universal Service cc Docket No. 9645, 13 FCC RD 11501, release Number 98-67 released April 10, 1998.

FCC Docket No. WC 03-45.

FCC 04-27 Petition for Declaratory Ruling that pulver.com’s Free World Dialup is Neither Telecommunications Nor a Telecommunications Service, released February 19, 2004.

FCC Docket No. WC 03-271.

FCC Docket No. WC 02-361.

www.888localdial.com.

In the Matter of Federal-State Joint Board on Universal Service cc Docket No. 9645, 13 FCC RD 11501, release Number 98-67 released April 10, 1998, Para. 88.

Id. Para. 91.

Id. Para. 39.

Id. Para. 39.

Id. Para. 52.

Id. Para. 59.

Id. Para. 89.

Case 01-C-1119 Complaint of Frontier Telephone of Rochester Against US DatNet Corporation Concerning Alleged Refusal to Pay Intrastate Carrier Access Charges, May 31, 2002.

In Matter of Federal-State Joint Board on Universal Service cc Docket No. 9645, 13 FCC RD 11501, release Number 98-67 released April 10, 1998.